# Supporting Information:

# Waste-to-Resource value chain optimisation: combining spatial, chemical and technoeconomic aspects

Ivan Roblesa, Edward O’Dwyera, Miao Guoa,b,*

Department of Chemical Engineering, Imperial College London, London SW7 2AZ UK

b Department of Engineering, King’s College London, London Strand Campus WC2R 2LS UK

* Corresponding author miao.guo@imperial.ac.uk, miao.guo@kcl.ac.uk

# Technology parameterisation

The technology parameterisation approach used in this work is outlined in this Supporting information document. Variables used in the succeeding analysis are summarised in Table S1.

Table S1: List of variables and parameters used

| Variables | Description | Units |
| --- | --- | --- |
| ADspecCPX | Specific capital cost of AD plant with power generation | (£M/MWOut) |
| ADspecOPX | Specific yearly operational cost of AD plant with power generation | (£M/MWOut) |
| BUPCPX | Capital cost of biogas upgrading facility | (£/therm/yr) |
| BUPOPX | Operational cost of biogas upgrading facility | (£/thermout) |
| capMWeAD | AD power plant power output capacity | (MW) |
| capkWeAD | AD power plant power output capacity | (kW) |
| captherm | Biogas upgrading plant output capacity | (therm) |
| COD | Chemical Oxygen demand | (kg) |
| frdry SF | Water fraction in digestate SF prior to drying | (kg/kg) |
| frH2O dry SF | Water fraction in digestate SF after drying | (kg/kg) |
| GSECPX | Capital cost of gas engine system | (£) |
| GSEOPX | Operation and maintenance cost of gas engine system | (p/kWh) |
| Hv | Heat of vaporisation of water | (MW/kg) |
| HHV | Higher heating value | (MJ/kg) |
| LHV | Lower heating value | (MJ/kg) |
| Indexyear | Cost index used to relate the equivalent cost in one year to another | (£/£) |
| INJCPX | Capital cost of gas injection to the grid system | (£/therm/yr) |
| INJOPX | Operation cost of gas injection to the grid system | (£/thermout) |
| M | Mass | (kg) |
| ORCCPX | Capital cost of Organic Rankine cycle | (£) |
| ORCOPX | Yearly operational cost of Organic Rankine cycle | (£/yr) |
| Sepx | Separation factor of digestate constituents into SF and LF | (kg/kg) |

## Anaerobic digestion and digestate preparation

In AD, OFMSW is converted to biogas, which can be further upgraded to produce bio-methane or processed to produce electricity. The plants considered in this model are AD+CHP and AD+CHP+BtG systems, with the relevant equations used to calculate the OPEX and CAPEX presented in Table S2. The ratio for the capacities of bio-methane to electricity production for AD+CHP+BtG is assumed to be 46.3 therm of bio-methane to 1 GJ of electricity, which is the average for this kind of plants in the UK (NNFCC, 2019). The electrical efficiency conversion for CHP was assumed as 1.2 kWh/m3 of biogas produced and the calorific value of biogas is 21.48 kJ/l (Guo et al., 2013).

Eq. (S1)-(S2) are used to estimate the costs of AD+CHP facilities. For the AD+CHP+BtG facilities, costs are broken down into gas engine (Eq. (S3)-(S4)), Organic Rankine Cycle (ORC) (Eq. (S5)-(S6)), biogas upgrade (Eq. (S7)-(S8)), and biogas grid injection (Eq. (S9)-Eq. (S10)) components. The equations in Table S2 were derived from data in the listed references and used to develop cost parameters for technologies corresponding to three different sizes used in the optimisation problem. These, along with the resulting annualised capital costs are summarised in Table S8.

Table S2: Equations for CAPEX and OPEX estimation of AD plants

| Description | Equation |  |  |  |
| --- | --- | --- | --- | --- |
| AD+CHP CAPEX1 | ADspecCPX=5.425capMWeAD-0.08 |  | (S1) |  |
| AD+CHP OPEX1 | ADspecOPX=0.319capMWeAD-0.22 |  | (S2) |  |
| Gas engine system CAPEX2 | GSECPX=3453capkWeAD0.666+11167 |  | (S3) |  |
| Gas engine system OPEX2 | GSEOPX=6.13capkWeAD-0.2219 |  | (S4) |  |
| Organic Rankine Cycle CAPEX2 | ORCCPX=1733capkweAD+11363 |  | (S5) |  |
| Organic Rankine Cycle OPEX2 | ORCOPX=57.82capkWeAD |  | (S6) |  |
| Biogas upgrading CAPEX3 | BUPCPX=16198captherm-0.82 |  | (S7) |  |
| Biogas upgrading OPEX3 | BUPOPX=19779captherm-0.767 |  | (S8) |  |
| Biogas grid injection CAPEX4 | INJCPX=413102captherm-1.03 |  | (S9) |  |
| Biogas grid injection OPEX4 | INJOPX=205.33captherm-0.472 |  | (S10) |  |
| 1(Sbodova, 2003), 2(Oreggioni et al., 2017), 3(Deng and Hägg, 2010), 4(IRENA, 2018) |  |  |  |

Energy recovery from digestate solid fraction requires digestate pre-treatment. In the case of incineration, the digestate must be dried and in the case of pyrolysis, it must be dried and pelletized (Peng and Pivato, 2019; WRAP, 2012). To model this, values reported in (Nagy et al., 2018) were used whereby digestate solid (SF) and liquid (LF) fractions are separated using a screw press. The SF is then dried using a belt drier and recycled heat from CHP, before being pelletized using a pelletiser. In Table S3, the unit product price for revenue calculation is presented. The relevant CAPEX and OPEX data are presented Table S8.

Table S3: Price of process products

| Product | Price (£/unit) |
| --- | --- |
| Electricity (MWh)1 | 58.23 |
| Biogas (therm)1 | 0.604 |
| Biochar (tonne)2 | 33.6 |
| Green SNF (GJ)3 | 11.52 |
| Bio Oil (gallon)4 | 2.56 |
| 1 (Ofgem, 2016), 2(Tan et al., 2014), 3(Haldor Topsøe, 2012), 4 (Brown et al., 2013) |  |

Note that all the capital costs in this model have been adjusted to the year 2018 using the CEPCI (Chemical Engineering, 2019) and the operational costs have been adjusted to account for inflation by using the producer price index (PPI) (OECD, 2019). Eq. (S11) is used to calculate the cost in the year of interest by using the relevant indices and cost during the original year.

| Cost2018=Index 2018Indexyear (Original)Costyear (Original) | (S11) |
| --- | --- |

## Incineration

To model the resource recovery from digestate by incineration, the extractable energy quantity must be modelled. To achieve this, the empirical relation between elemental species and the Higher Heating Value (HHV) for biomass provided in (Acar et al., 2011) is used and shown in Eq. (S12), where MC, MH, MO, MN and Mfuel are the masses of carbon, hydrogen, oxygen, nitrogen and the fuel respectively. Furthermore, the Lower Heating Value (LHV) is calculated from the HHV as shown in Eq. (S13) (Li et al., 2017), where MH2O is the water mass and the latent heat of vaporisation Hv=2.26 MJ kg-1 (Datt, 2011).

| HHV=33.5 MCMfuel+142.3 MHMfuel-15.4 MOMfuel-24.5 MNMfuel | (S12) |
| --- | --- |
| LHV=HHV-HvMH2OMfuel | (S13) |

Incineration of biomass wastes utilise complex Rankine steam cycle configurations with electrical efficiencies of 28% (Tobiasen et al., 2014) used in the model.

## Pyrolysis

During the pyrolysis of digestate, syngas, oil and char are produced. The yields reported by (Wei et al., 2019) were followed in the model, where on a dry basis 25.6%wt is syngas, 8.6 wt% is bio oil and 42% is char. The calorific value of syngas was assumed to be 11MJ kg-1 (Park et al., 2015). All operational costs related to pyrolysis are considered except that of the feedstock, as this is obtained as waste from AD.

## Waste feed chemical characterisation

The spatially-explicit OFMSW resource potential was derived from (WDF, 2019). UK OFMSW can be broadly classified into four categories: (i) food waste, (ii) green garden waste, (iii) mixed garden/food waste and (iv) other compostable waste. The compositions for these wastes are shown in Table S4, and used in the optimisation model as parameters.

For the elemental analyses, the composition of hemicellulose is taken as that of Xylan and the ‘other compostable waste’ category is characterised as generic mechanically recovered OFMSW.  When considering mixed food and garden waste, a ratio of 1.21:1 respectively is assumed based on typical MSW sample fractions recorded by (WastesWork and AEA, 2010). Additionally, for the AD of garden waste, lignin is assumed to not be anaerobically digestable,  and remains in the digestate after AD processing (Antoniou et al., 2019). Even though this strongly inhibits the potential for biogas production, it increases the possibilities for further resource recovery through the posttreatments mentioned above.

Table S4: Elemental analysis of the digestible part of the relevant types of OFMSW and component analysis of garden waste

| Element | Food waste (% TS)1 | Garden waste (%TS*)2 | Mr-OFMSW (%TS)3 |
| --- | --- | --- | --- |
| Nitrogen | 3.63 | 1.65 | 1.32 |
| Carbon | 47.54 | 26.61 | 33.0 |
| Hydrogen | 6.40 | 3.96 | 4.80 |
| Oxygen | 36.10 | 34.21 | 22.2 |
| Component |  | (%TS) |  |
| Lignin | - | 25.68 | - |
| Cellulose | - | 38.54 | - |
| Hemicellulose | - | 26.24 | - |
| 1 (Esteves and Devlin, 2010), 2 (Bhange et al., 2015), 3 (Zhang et al., 2012) *Considers the digestable components, i.e. lignin is not included |  |  |  |

The theoretical biogas production (with an assumed composition of 65% CH4 and 35% CO2 (v/v)) was estimated from the chemical oxygen demand (COD), calculated from Eq. (S14) (Patel et al., 2016). For a given chemical species with elemental composition of CnHaObNc (1 mol).

| COD=2n+(a-3×c)/2-b232g O2 | (S14) |
| --- | --- |

Each  mole of CH4 consumes two moles of oxygen, implying that 1g COD destruction is equivalent to 0.35L CH4 at 0°C and 760mm Hg pressure (STP) or 0.395L CH4 at 35°C and one atmosphere (Speece, 1996). The results for COD present in the waste influents and the biogas produced from these are presented in Table S5.

Table S5: Density, TS% and calculated COD and biogas production for the pertinent inflows

| Waste | Density (kg m-3) | TS/TDS (%) | COD (g L-1) | COD (g kg-1) | Biogas (L kg-1) |
| --- | --- | --- | --- | --- | --- |
| Food | 811.601 | 25.7 3 | 267.0 | 329.2 | 170.1 |
| Garden | 379.70 1 | 44.844/29.05 | 73.62 | 194.3 | 100.3 |
| Food and garden | 480.90 | 28.73/26.23 | 230.6 | 310.5 | 160.4 |
| Mr-OFMSW | 722 2 | 52.835 | 315.0 | 436.3 | 225.4 |
| 1 (US EPA, 2014), 2 (Mata-Alvarez et al., 2000), 3 (Esteves and Devlin, 2010), 4 (Chester et al., 2008), 5 (Zhang et al., 2012), TS: total solids; TDS: total digestible solids |  |  |  |  |  |

## Digestate product chemical characterisation

To characterise the digestate, a mass balance is made in terms of the elemental analysis. Following the conversion of COD into biogas, the mass (M) of elements required to produce this species are removed from the influent to produce the gas phase. The remaining species are balanced as digestate. In this manner Eq. (S15) and (S16) are used to model the remaining elemental constituents (x) in each stream.

| Mbiogas,x=CODconsumedyldxbp | (S15) |
| --- | --- |
| Mdigestate,xWD=Mwaste,x-Mbiogas,x | (S16) |

The ratios for methane production per gram of COD and biogas composition are used to determine the yields (yldxbp) of these elemental species in the biogas phase, assuming that both CO2 and CH4 behave as ideal gases. The resulting values are summarised in Table S6.

Table S6: Elemental yield of species in the biogas phase

| Component | Carbon | Hydrogen | Oxygen |
| --- | --- | --- | --- |
| yldxbp (g/g COD) | 0.2451 | 0.0531 | 0.2288 |

Furthermore, the digestate is divided into two separate fractions, the solid fraction and the liquid fraction. The amount of nitrogen that goes to each is 87% and 13%  LF and SF respectively (Tambone et al., 2017).  The ratio of separation  for the rest of the solid species is 1:12 for SF and LF respectively  (Drosg et al., 2015).  These separation factors (sepx) are summarised in Table S7.

Typical TS contents of the SF of digestate of OFMSW ranges at about 17% (Peng and Pivato, 2019) but to make further processing of digestate by incineration feasible it must be dried, while for pyrolysis, TS content must be increased to 80% and pelletised (Peng and Pivato, 2019; WRAP, 2012). Therefore, the fraction of water in the dry SF of the digestate (frH2O dry SF)will be that required to maintain the aforementioned percentages. The amount of water assigned to the LF will be that not assigned to the SF before its water content is decreased, where (frH2O SF) is the fraction of water in SF digestate prior to drying. Eq. (S17)-(S21) show the mass balances, with the water fractions summarised in Table S7.

| MxSF=Mdigestate,spsepx , x∈{N,C,H,O,lignin} | (S17) |
| --- | --- |
| MxLF=Mdigestate,sp1-sepx, x∈{N,C,H,O,lignin} | (S18) |
| MH2OSF=MSF digestatefrH2O dry SF | (S19) |
| MH2OLF=MH2OWF-(MSF digestatefrH2O SF) | (S20) |
| MdigestateF=x∈XMxF , F∈{SF,LF} | (S21) |

Table S7: Composition parameters for solid and liquid digestate fractions

| Parameter (g/g) | C/H/O/lignin | Nitrogen | H2O |
| --- | --- | --- | --- |
| distx | 0.923 | 0.130 | - |
| frH2O SF | - | - | 0.83 |
| frH2O dry SF | - | - | 0.20 |

## Technology parameterisation

The selection of a suitable national-level approach requires the characterisation of the various technologies, processes, and waste and product streams considered. To represent these system components, an extensive parameterisation is carried out and summarised in the SI. Some of the most relevant capital (CAPEX) and operational (OPEX) expenditure data emerging from this are presented in Table S8.

Table S8: Parameters for cost evaluation of the relevant technologies

| Technology | Capital investment | Life | Annualised capital cost | Operational cost |  | Capacity |
| --- | --- | --- | --- | --- | --- | --- |
|  | (£) | (years) | (£/years) | (£/GJ) |  | (kW) |
| AD CHP (small) 1,2 | 1593631 | 15 | 106242 | 13.89 |  | 250 |
| AD CHP (medium) 1,2 | 11544834 | 20 | 769656 | 8.37 |  | 2500 |
| AD CHP (large) 1,2 | 31391834 | 20 | 1569592 | 6.48 |  | 8000 |
|  | (£) | (years) | (£/years) | (£/GJ)+ | (£/GJ)++ | (kW) |
| AD CHP & BtG (small) 1,2,3 | 2761201 | 15 | 184080 | 14.72 | 25.1 | 250 |
| AD CHP & BtG (medium) 1,2,3 | 16071236 | 20 | 803562 | 8.87 | 6.07 | 2500 |
| AD CHP & BtG (large) 1,2.3 | 46686529 | 20 | 2334326 | 6.86 | 3.03 | 8000 |
|  | (£) | (years) | (£/years) | (£/tonne) |  | (tonnes/year) |
| Incineration4 | 45000000 | 25 | 1800000 | 40.00* |  | 150000* |
| Pyrolysis5,6 | 8661322 | 25 | 346453 | 50.32* |  | 35989* |
| Drying7 | 283000 | 10 | 28300 | 23.13** |  | 3500** |
| Drying & pelletisation7 | 340518 | 10 | 34052 | 74.77** |  | 3500** |
| Note: an exchange ratio of 1.25 US$/£ and 1.12 €/£ are used to calculate the costs from the original sources * Per tonne of feed, ** per dry tonne of product + produced in the CHP system of the plant, ++Produced in the BtG system of the plant, CHP capacity for CHP & BtG systems are the same as CHP systems only 1(Oreggioni et al., 2017), 2(Sbodova, 2003),3(Deng and Hägg, 2010),4(Mckendry, 2018),5(Brown et al., 2013),6(Wang et al., 2018) ,7 (Nagy et al., 2018) |  |  |  |  |  |  |  |

# References

Acar, S., Ayanoglu, A., Demirbas, A., 2011. DETERMINATION OF HIGHER HEATING VALUES (HHVS) OF BIOMASS FUELS. FUELS AND COMBUSTION IN ENGINEERING JOURNAL 119, 14–16.

Antoniou, N., Monlau, F., Sambusiti, C., Ficara, E., Barakat, A., Zabaniotou, A., 2019. Contribution to Circular Economy options of mixed agricultural wastes management: Coupling anaerobic digestion with gasification for enhanced energy and material recovery. Journal of Cleaner Production 209, 505–514. https://doi.org/10.1016/j.jclepro.2018.10.055

Bhange, V.P., William, S.P., Sharma, A., Gabhane, J., Vaidya, A.N., Wate, S.R., 2015. Pretreatment of garden biomass using Fenton’s reagent: Influence of Fe2+ and H2O2 concentrations on lignocellulose degradation. Journal of Environmental Health Science and Engineering 13. https://doi.org/10.1186/s40201-015-0167-1

Brown, T.R., Thilakaratne, R., Brown, R.C., Hu, G., 2013. Techno-economic analysis of biomass to transportation fuels and electricity via fast pyrolysis and hydroprocessing. Fuel 106, 463–469. https://doi.org/10.1016/j.fuel.2012.11.029

Chemical Engineering, 2019. Economic Indicators. Chemical Engineering essentiaks for the CPI professional, Available at: https://www.chemengonline.com/pci (Accessed: 10 July 2019)

Chester, M., Stupples, D., Lees, M., 2008. A Comparison of the Physical and Chemical Composition of Uk Waste Streams Based on Hypothetical Compound Structure. 13th European Biosolids and Organic Resources Conferencee, 10- 12 November 17. https://doi.org/10.1186/1742-7622-5-2

Datt, P., 2011. Latent Heat of Vaporization/Condensation, in: Singh, V.P., Singh, P., Haritashya, U.K. (Eds.), Encyclopedia of Snow, Ice and Glaciers. Springer Netherlands, Dordrecht, p. 703. https://doi.org/10.1007/978-90-481-2642-2_327

Deng, L., Hägg, M.B., 2010. Techno-economic evaluation of biogas upgrading process using CO 2 facilitated transport membrane. International Journal of Greenhouse Gas Control 4, 638–646. https://doi.org/10.1016/j.ijggc.2009.12.013

Drosg, B., Fuchs, W., Al, T., Madsen, S.M., Linke, B., 2015. Nutrient Recovery by Biogas Digestate Processing Bernhard, IEA Bioenergy.

Esteves, S., Devlin, D., 2010. Food Waste Chemical Analysis Chemical. wrap Material change for a better environment.

Guo, M., Stuckey, D.C., Murphy, R.J., 2013. Is it possible to develop biopolymer production systems independent of fossil fuels? Case study in energy profiling of polyhydroxybutyrate-valerate (PHBV). Green Chemistry 15, 706–717. https://doi.org/10.1039/c2gc36546d

Haldor Topsøe, 2012. SOEC Electrochemical upgrading of biogas to pipeline quality by means of SOEC electrolysis.

IRENA, 2018. Biogas for Road Vehicles: tehnology brief.

Li, H., Mehmood, D., Thorin, E., Yu, Z., 2017. Biomethane Production Via Anaerobic Digestion and Biomass Gasification. Energy Procedia 105, 1172–1177. https://doi.org/10.1016/j.egypro.2017.03.490

Mata-Alvarez, J., Macé, S., Llabrés, P., 2000. Anaerobic digestion of organic solid wastes. An overview of research achievements and perspectives. Bioresource Technology 74, 3–16. https://doi.org/10.1016/S0960-8524(00)00023-7

Mckendry, P., 2018. Costs of Incineration and Non-Incineration Energy from Waste Technologies Energy from Waste Technologies, Green Acre Consulting. SLR, Greater London Authority. https://doi.org/10.13140/RG.2.2.30993.12649

Nagy, D., Balogh, P., Gabnai, Z., Popp, J., Oláh, J., Bai, A., 2018. Economic analysis of pellet production in co-digestion biogas plants. Energies 11, 1–21. https://doi.org/10.3390/en11051135

NNFCC, 2019. AD portal map site list external May 2019, NNFCC, Available at: http://www.biogas-info.co.uk/ad-portal-map_site-list_external_may-_2019/ (Accessed: 5 July 2019)

OECD, 2019. Producer price indices (PPI). OECD Data, Available at: https://data.oecd.org/price/producer-price-indices-ppi.htm (Accessed: 11 July 2019)

Ofgem, 2019. Monitoring the market: wholesale market indicators. Available at: https://www.ofgem.gov.uk/data-portal/wholesale-market-indicators (Accessed: 18 June 2019).

Oreggioni, G.D., Gowreesunker, B.L., Tassou, S.A., Bianchi, G., Reilly, M., Id, M.E.K., Toop, T.A., Theodorou, M.K., 2017. Potential for Energy Production from Farm Wastes Using Anaerobic Digestion in the UK : An Economic Comparison of Different Size Plants. Energies 1–16. https://doi.org/10.3390/en10091396

Park, I., Jeong, S., Obokata, T., Okada, H., Wafers, G., Tanaka, T., Takamiya, S., 2015. Simulation of a process for the two-stage thermal conversion of biomass into the synthesis gas. Journal of Physics: Conference Series 653. https://doi.org/10.1088/1742-6596/653/1/012031

Patel, B., Guo, M., Shah, N., Hellgardt, K., 2016. Environmental profile of algal Hydrothermal Liquefaction - A country specific case study. Algal Research 16, 127–140. https://doi.org/10.1016/j.algal.2015.12.017

Peng, W., Pivato, A., 2019. Sustainable Management of Digestate from the Organic Fraction of Municipal Solid Waste and Food Waste Under the Concepts of Back to Earth Alternatives and Circular Economy. Waste and Biomass Valorization 10, 465–481. https://doi.org/10.1007/s12649-017-0071-2

Sbodova, I., 2003. Anaerobic digestion , storage , oligolysis , lime , heat and aerobic treatment of livestock manures Final report.

Speece, R.E., 1996. Anaerobic Biotechnology for Industrial Wastewaters, in: Anaerobic Biotechnology for Industrial Wastewaters,. Archae Press, Nashville, Tennessee, p. 53.

Tambone, F., Orzi, V., D’Imporzano, G., Adani, F., 2017. Solid and liquid fractionation of digestate: Mass balance, chemical characterization, and agronomic and environmental value. Bioresource Technology 243, 1251–1256. https://doi.org/10.1016/j.biortech.2017.07.130

Tan, E.C.D., Marker, T.L., Roberts, M.J., 2014. Direct production of gasoline and diesel fuels from biomass via integrated hydropyrolysis and hydroconversion process—A techno-economic analysis. Environmental Progress & Sustainable Energy 33, 609–617. https://doi.org/10.1002/ep.11791

Tobiasen, L., Kahle, K., Hindsgaul, C., Kamuk, B., 2014. Waste to Energy – Energy Recovery of Green Bin Waste: Incineration/Biogas Comparison. Current Sustainable/Renewable Energy Reports 1, 136–149. https://doi.org/10.1007/s40518-014-0018-1

US EPA, 2014. Volume-to-Weight Conversion Factors.

Wang, X., Guo, M., Koppelaar, R.H.E.M., Van Dam, K.H., Triantafyllidis, C.P., Shah, N., 2018. A Nexus Approach for Sustainable Urban Energy-Water-Waste Systems Planning and Operation. Environmental Science and Technology 52, 3257–3266. https://doi.org/10.1021/acs.est.7b04659

WastesWork, AEA, 2010. The composition of municipal solid waste in Wales. Wrap 38–39.

WDF, 2019. Data on organic fraction municipal solid waste. Waste Data Flow, Available at: http://www.wastedataflow.org/ (Accessed: 5 June 2019)

Wei, Y., Tang, J., Xie, J., Shen, C., 2019. Molten alkali carbonates pyrolysis of digestate for phenolic productions. Journal of Cleaner Production 225, 143–151. https://doi.org/10.1016/j.jclepro.2019.03.234

WRAP, 2012. Enhancement and treatment of digestates from anaerobic digestion. Waste and Resources Action Programme 1–38. https://doi.org/10.1007/s11666-009-9304-7

Zhang, Y., Banks, C.J., Heaven, S., 2012. Anaerobic digestion of two biodegradable municipal waste streams. Journal of Environmental Management 104, 166–174. https://doi.org/10.1016/j.jenvman.2012.03.043