# Engineering strategies for producing medium-long chain dicarboxylic acids in oleaginous yeasts

# Abstract

Medium-long chain dicarboxylic acids (DCAs, C6) are essential chemical raw materials, with wide applications in the chemical, pharmaceutical, material and food industries. However, the traditional chemical synthesis methods cause environmental pollution and are not in line with goals of sustainable development. With the development of synthetic biology, high-value-added DCAs can be biosynthesized from hydrophobic substrates (HSs) using suitable microorganisms. This review first summarizes the biosynthetic pathway of DCAs in oleaginous yeasts and then emphasizes the related engineering strategies for increasing the product yield, including promoter, enzyme, pathway, cell, fermentation, and downstream engineering. In addition, the challenges and development trends in the biosynthesis of DCAs are discussed, in light of the current progress, challenges, and trends in this field. Finally, guidelines for future research are proposed. Overall, this review systematically summarizes recent engineering strategies for DCAs production in oleaginous yeasts and offers valuable insights for future DCAs biosynthesis.

Keywords: Hydrophobic substrates; Synthetic biology; ω-Oxidation; Yarrowia lipolytica

- Introduction

Medium-long chain dicarboxylic acids (DCAs) are generally defined as containing 6 or more atoms in the carbon chain, with carboxylic groups located on the α and  positions. DCAs can be utilized as raw materials for preparing many high-value-added products, such as the nylon-type polyamide polymers, serving as platform compounds for the chemical, food, textile, pesticide and pharmaceutical industries (Li et al., 2020; Sathesh-Prabu and Lee, 2015; Wan and Lee, 2021). For example, adipic acid (ADA), an aliphatic C6 DCA, is mostly used for manufacturing nylon-6,6, polyvinyl chloride and polyvinyl butyral (Xu et al., 2023). Azelaic acid (AZA) possesses anti-inflammatory and antibacterial properties, making it a commonly used ingredient in pharmacological products (Hung et al., 2021). Sebacic acid (SA) can be used not only for the manufacture of polyamide polymers (e.g. nylon-6,10, nylon-4,10, nylon-10,10), but also in the production of lubricants (Jeon et al., 2019).

Moreover, the market demand for DCAs is increasing steadily. It is reported that the production of adipic acid is growing at a rate of 3-5% annually, with the global market is projected to exceed $8 billion by 2025 (Rios et al., 2021). However, DCAs are still produced almost exclusively through chemical methods. For example, adipic acid is synthesized via the chemical oxidation of cyclohexane (Bart and Cavallaro, 2015). Azelaic acid can be synthesized using a traditional method that involves the oxidation of oleic acid with ozone, resulting in the production of nonanoic acid as a by-product (Kim et al., 2019). Sebacic acid is produced by the alkaline lysis of castor oil and its derivatives (Yu et al., 2020). However, these traditional chemical methods have significant limitations, including a restricted range of accessible carbon chain lengths, numerous complex conversion steps, high energy requirements, environmental pollution, and the generation of toxic by-products.

Given the increasing demand for DCAs and the need to minimize their adverse environmental effects, substantial efforts are needed to develop more sustainable production processes. Fortunately, the development of metabolic engineering and the emerging synthetic biology technologies offers an innovative solution for the sustainable production of DCAs (Chae et al., 2020; Kim et al., 2023; Ko et al., 2020; Lawson et al., 2021; Zhou et al., 2024). While many biomanufacturing processes primarily rely on hydrophilic substrates as the key sources of carbon and energy to produce high-value compounds, hydrophobic substrates (HSs) are more advantageous carbon sources for DCAs biosynthesis. Table 1 provides a detailed comparison of the production of DCAs from different carbon sources. Oleaginous yeasts such as Candida sp. and Yarrowia lipolytica have a remarkable ability to assimilate HSs, which makes them a suitable host for the production of DCAs via the unique -oxidation pathway recognized as a subsidiary route of fatty acid β-oxidation (Werner and Zibek, 2017). The majority of recent studies on high-yield production focused on n-alkanes (Lee et al., 2019). However, alkanes are primarily derived from increasingly depleting fossil resources, making them unsuitable for the sustainable production of DCAs. With sustainable development becoming a key focus on the global agenda, utilizing readily available waste vegetable oils or animal fats as feedstocks in biomanufacturing could promote a circular bioeconomy. Fatty acids (FAs) are naturally generated by plants, while fatty acid methyl esters (FAMEs) are generated through the transesterification of vegetable oils (Berchmans and Hirata, 2008; Fukuda et al., 2001). Researchers are now exploring alternative hydrophobic substrates, such as renewable FAs and FAMEs (Ju et al., 2020; Lu et al., 2010). Although the conversion of alkanes is still the prevailing method, the emergence of novel approaches offers hope for the sustainable production of DCAs. Figure 1 shows an overview of the production of DCAs from different carbon sources and their applications. Table 2 provides a summary of crucial performance metrics for DCAs production, such as the chassis strain and substrates, titer, productivity, and yield, along with corresponding engineering strategies.

Here, this review summarizes recent advances in the production of DCAs from hydrophobic substrates by oleaginous yeasts. Firstly, the biosynthetic pathways are reviewed, with emphasis on α-, ω-, and β-oxidation pathways. The engineering strategies for increasing DCAs production are then introduced in detail, including promoter, enzyme, pathway, cell, fermentation, and downstream engineering. Finally, possible future developments in DCAs biosynthesis are prospected, which promise to overcome the main obstacles encountered in DCAs production.

- Metabolic pathways of dicarboxylic acids

In the next three parts, the metabolic processes of DCAs production in oleaginous yeasts are described in detail. The metabolic processes discussed here include the conversion of n-alkanes into FAs by α-oxidation, which is followed by the conversion of FAs into DCAs by -oxidation, and finally the degradation of both FAs and DCAs by -oxidation. In addition, oleaginous yeasts can directly hydrolyze FAMEs to release free FAs using an efficient lipase-expressing system (Lee et al., 2024), which then enter the cell for -oxidation (Figure 2).

## 2.1. Alpha-oxidation pathway

During primary oxidation, n-alkanes are transformed into FAs of identical chain-length within the endoplasmic reticulum (ER) or peroxisomes (Figure 2). Initially, n-alkanes undergo α-oxidation in the presence of a hydroxylase complex to produce fatty alcohols. This complex includes a cytochrome P450 monooxygenase (CYP/Alk) and a NADPH-dependent cytochrome P450 reductase (Cpr), both embedded in the ER membrane (Nebert et al., 1991). Notably, this reaction serves as the slowest step in the n-alkane oxidation process. CYPs belonging to the CYP52 family are encoded by different genes in form of isoenzymes in a single organism, (Alk1-Alk12) as was shown in Y. lipolytica (Hirakawa et al., 2009; Takai et al., 2012) and C. tropicalis (CYP52A12-CYP52A20, CYP52D2) (Craft et al., 2003). CYP52-family P450s from Y. lipolytica show interesting substrate specificities. Studies have shown that Alk1 is mainly involved in the metabolism of medium-chain alkanes (such as n-decane), while Alk2 prefers long-chain alkanes (such as n-hexadecane) in Y. lipolytica (Iida et al., 2000). On the other hand, Cpr, which is a flavin protein including both flavin adenine dinucleotide (FAD) and flavin mononucleotide (FMN) binding domains, primarily facilitates electron transfer (Folz et al., 2023). Cpr exhibits a strong preference for NADPH (Elmore and Porter, 2002), which also implies that Cpr-mediated catalysis needs to be supported by cellular metabolism or supplementation for in vitro applications.

The fatty alcohol produced by the CYP and Cpr undergoes further oxidation to the corresponding aldehyde in the ER by a NAD(P)+-dependent fatty alcohol dehydrogenase (Adh). Additionally, fatty alcohol oxidase (Fao) predominantly catalyses the oxidation of long-chain alkanols, this step occurs in the peroxisome and produces H2O2. (Ilchenko et al., 1994; Mauersberger et al., 1992). Adh is generally considered to be more active with NADH as a cofactor (Aziz et al., 2016). Iwama et al. found that a mutant of Y. lipolytica with the deletion of eight ADH genes and a FAO gene were difficult to grow on fatty alcohols (Iwama et al., 2015). Among them, ADH1, ADH3 and FAO1 were found to play essential roles in the oxidation process of exogenous fatty alcohols. In addition, this research demonstrated that Adh1p and Adh3p are cytosolic while Fao1p is localized to peroxisomes.

Finally, an NAD(P)+-dependent fatty aldehyde dehydrogenase (Faldh), which is present in the ER or peroxisomes, catalyzes the conversion of fatty-aldehydes into FAs (Iwama et al., 2014). Faldh uses NAD(P)+ as a cofactor, and most of its variants show a preference for NAD+ as an electron acceptor (Bertram et al., 2017). The Faldh enzymes of Y. lipolytica that are active during n-alkane metabolism are encoded by HFD1-4 that are four paralogous genes. Iwama et al. (Iwama et al., 2014) constructed a mutant of Y. lipolytica with the deletion of four genes encoding Faldh and found that it could no longer use alkanes with a chain length of 12-18 for growth, but expressing any one of these HFD genes in the mutant restored its growth on n-alkanes. Therefore, Faldh deserves our attention as a key player in the metabolic process of n-alkanes.

## 2.2. Omega-oxidation pathway

It is generally accepted that FAs are converted into DCAs through -oxidation in the ER, which consists of three successive oxidation steps similar to α-oxidation (Figure 2). In the oxidation of FA, ω-oxidation is a complementary pathway of β-oxidation, which can make up for β-oxidation disorder to a certain extent (Folz et al., 2023; Ranea-Robles and Houten, 2023).

The enzymes involved in primary alkane oxidation and ω-oxidation are likely the same. In the ω-oxidation process, FAs are initially oxidized at the terminal carbon to form ω-hydroxy fatty acids (ω-HFAs). This reaction is catalyzed by the ω-hydroxylase complex consisting of CYP/Alk and Cpr. In Y. lipolytica, Alks exhibit varied substrate specificities and physiological functions in the oxidation of alkanes and FAs. Studies have shown that ALK3, ALK5 and ALK7 encode crucial enzymes that hydroxylate the ω-terminal end of dodecanoic acid, while enzymes encoded by other ALK genes only showed low activity (Iwama et al., 2016). It is equally important that Cpr supplies electrons required to produce ω-HFAs.

In the second oxidation step, ω-HFAs are converted into corresponding fatty acid aldehydes, facilitated primarily by Adh or Fao within the ER and peroxisomes, respectively. Gatter (2014) deleted the fatty alcohol dehydrogenase genes (FADH, ADH1-7) and the fatty alcohol oxidase gene (FAO1) in Y. lipolytica lacking β-oxidation, and used n-alkanes or FAs as the substrate. While the eight ADH genes were deleted, it resulted a modest increase of ω-HFA accumulation. And the removal of the FAO1 gene showed a more significant effect. Additionally, simultaneous deletion of ADH and FAO1 further decreased DCAs production. These findings suggest that both Adh and Fao participate in the ω-oxidation of FAs, with Fao playing a more critical role, marking this as the rate-limiting step of DCAs production.

The final step of ω-oxidation is the conversion of the aldehyde into a carboxyl group to form DCAs, catalyzed by Faldh (Kim et al., 2020b; Lim et al., 2021; Seo et al., 2018). Faldh is present in the ER and peroxisomes, which also indicates that DCAs may be produced in both of these compartments. However, it is generally accepted that DCAs are produced in the ER.

## 2.3. Beta-oxidation pathway

In mammals, β-oxidation mainly occurs in the mitochondria and peroxisomes (Valm et al., 2017). However, in some lower eukaryotes, such as Y. lipolytica and C. tropicalis, β-oxidation occurs in peroxisomes (Kurihara et al., 1992; Liu et al., 2022; Zhao et al., 2023). FAs/DCAs need to be activated via an acyl-CoA synthetase (Acs) before entering cyclical β-oxidation (Figure 2). The β-oxidation process produces an acetyl-CoA and an acyl-CoA with two carbon atoms missing through a four-step reaction, which is catalyzed by acyl-CoA oxidase (Pox), 3-ketoacyl-CoA thiolase (Pot) and a multifunctional enzyme (Mfe) (Yu et al., 2023).

In this pathway, Pox initiates β-oxidation by catalyzing its rate-limiting step (Morais et al., 2007). Pox converts the acyl-CoA ester into trans-2-enoyl-CoA while simultaneously generating hydrogen peroxide, which is then detoxified by catalase (Chen et al., 2023). Y. lipolytica possesses six acyl-CoA oxidases (Pox1–6), each with a preference for different chain lengths (Haddouche et al., 2010; Wang et al., 1999b). Among them, Pox1/6 exhibit no observable activity, Pox2 is more active on long-chain FAs, Pox3 prefers short-chain FAs, while Pox4/5 exhibit weak activity on a wide range of substrates (Guo et al., 2012; Waché et al., 2002; Wang et al., 1999a).

The subsequent two steps are facilitated by the Mfe, serving dual functions as an enoyl-CoA hydratase and a β-hydroxyacyl-CoA dehydrogenase, resulting in the formation of β-hydroxyacyl-CoA and β-ketoacyl-CoA, respectively (Petkevicius et al., 2022). In the concluding step of the β-oxidation pathway, the enzyme Pot catalyzed the reaction, generating an acetyl-CoA and an acyl-CoA which is shortened by a C2 unit (Berninger et al., 1993). This shortened acyl-CoA ester can undergo further cycles until it is completely degraded into acetyl-CoA. This acetyl-CoA can then be transported out of the peroxisome via a carnitine shuttle mechanism. In this process, carnitine-acetyltransferase (Cat) reversibly transfers the acetyl unit from acetyl-CoA to the carrier carnitine, and the acetyl-carnitine that is formed crosses the mitochondrial membrane to participate in the TCA cycle, where it contributes to ATP production (Messina et al., 2023; Strijbis and Distel, 2010).

- Engineering strategies for increasing the yield of dicarboxylic acids

In order to increase the production of DCAs in microorganisms, a number of novel engineering strategies are worth promoting. The following is an overview of common engineering strategies for DCAs production (Figure 3).

  - Promoter engineering

Promoters with dynamic transcriptional capacities or ideal characteristics are a precious tool in synthetic biology. Promoter engineering focuses on precise control of gene expression to optimize biosynthetic pathways, which in turn can increase DCAs production (Figure 3A).

In any host organism, employing strong and adjustable promoters is crucial for accurately regulating gene expression related to specific biosynthetic pathways. Several strategies for promoter engineering have been developed to increase the strength and flexibility of existing promoters. One such approach, hybrid promoter engineering, involves segmenting promoters into several components (including the core promoter, TATA box, and UAS sequences), which can then be recombined or duplicated to create a promoter with enhanced or adjustable activity (Lai et al., 2023; Xiong and Chen, 2020; Ye et al., 2024). Interestingly, introns can also have a positive effect on promoter activity (Gallegos and Rose, 2015; Hong et al., 2012). Additionally, the use of inducible promoters with designed transcription factor (TF) binding sites allows for expression to be triggered under specific conditions, thus making it possible for the effective separation of growth and product synthesis during fermentation (Cazier and Blazeck, 2021). In Y. lipolytica, the transcription of ALK1 is upregulated in response to alkane availability through a basic helix-loop-helix TF complex, composed of Yas1 and Yas2. Conversely, it is downregulated through Yas3, an Opi1-family transcription repressor (Endoh-Yamagami et al., 2007; Fukuda, 2013; Yamagami et al., 2004). Yas1 and Yas2 are constantly localized in the nucleus, binding to alkane-responsive element 1 (ARE1) in the ALK1 promoter (Kobayashi et al., 2013). In the absence of n-alkanes, Yas3 moves to the nucleus, binding to Yas2, and repressing ALK1 transcription. Conversely, the presence of n-alkanes induces Yas3 to remain in the ER, allowing ALK1 transcription to be activated (Hirakawa et al., 2009). Gaining a deeper insight into these TFs will aid in effectively controlling DCAs production under optimal conditions. Sun et al. provide a detailed report on promoter engineering in Y. lipolytica, which provides a theoretical basis for Y. lipolytica to become a superior biomanufacturing platform for producing DCAs (Sun et al., 2022).

The above promoter engineering strategies have been used in the production of DCAs. Wang et al. (2018) replaced the promoters of cytochrome P450 and NADPH-cytochrome P450 reductase with the constitutively expressed promoter pGAP in Candida tropicalis, the yield of long-chain dicarboxylic acids significantly increased to 32.84 g/L, which was 11.40 times that of the original strain. Park et al. (2021) constructed synthetic FadR promoters in Y. lipolytica, it can decouple cell growth from product synthesis. These promoters activated the ω-hydroxylating pathway, resulting in titers of ω-hydroxy FAs concentrations reaching as high as 160.00 mg/L. In addition, researchers used the oxygen-dependent promoter Pnar to achieve high production of DCAs in Escherichia coli under aerobic-anaerobic shift cultivation (Yu et al., 2017). These successful examples of inducible promoters can provide a reference for high-yield DCAs production from HSs in oleaginous yeasts. However, most current studies mainly focus on simple promoter replacement, and subsequent studies should apply more promoter engineering strategies to the production of DCAs.

  - Enzyme engineering

The essence of DCAs biosynthesis is the conversion of substrates into DCAs using the intracellular enzymes of microorganisms. In this process, considerations such as enzyme activity, selectivity, and expression pattern are crucial. Therefore, further engineering of enzymes provides an effective strategy to enhance DCAs production by modifying the characteristics of metabolic nodes within the DCAs synthesis pathways (Figure 3B).

Firstly, new enzymes can be identified and designed. More specifically, computer technology can be used to identify key enzymes in the DCAs synthesis pathway and reconstitute them in a rational way (Bell et al., 2024; Xie and Warshel, 2023). In the initial step of the ω-oxidation, the combined action of Alk and Cpr is usually required. If a new enzyme with the functions of these two enzymes can be discovered or developed, it may provide a new idea for the improvement of ω-oxidation activity.

Secondly, while optimizing enzyme screening and expression can help resolve challenges like low catalytic efficiency or limited stability and specificity. However, to further improve biocatalyst performance, additional protein engineering approaches, such as directed evolution and semi-rational or rational design, are necessary. Directed evolution simulates the natural evolutionary process of enzymes in a laboratory setting (Victorino da Silva Amatto et al., 2022). Typically, common methods used by researchers such as DNA shuffling, error-prone PCR, or the staggered extension process (StEP) to generate random mutations and construct libraries of mutants, after which high-throughput screening is employed to select enzymes with enhanced activity and functionality (Gargiulo and Soumillion, 2021; Markel et al., 2020; Packer and Liu, 2015; Victorino da Silva Amatto et al., 2022; Ye et al., 2018). By contrast, semi-rational or rational design employs a combination of synthetic biology and bioinformatics to pinpoint the enzyme's active site, offering a more targeted approach for enzyme engineering (Ali et al., 2020; Sequeiros-Borja et al., 2021). Meinhold et al. (2006) obtained a mutant that exhibited 52% selectivity for octane terminal positions by performing saturated mutagenesis on selected residues in the active site of cytochrome P450. This regional selectivity was specific to octane and did not transfer to other substrates.

In the biosynthetic pathway of DCAs, chain length specificity of key enzymes is very important, as it can greatly promote the conversion of substrates into target DCAs rather than various by-products. For example, the genome of Y. lipolytica encodes six acyl-CoA oxidases (Pox1-6) and twelve cytochrome P450 monooxygenases (Alk1-Alk12) related to DCAs synthesis (Bigey et al., 2023; Zhang et al., 2023). They all have different substrate specificity and physiological roles, which also enables Y. lipolytica to specifically synthesize target DCAs to a certain extent, but the mechanisms of action and preferred chain lengths of these enzymes are still unclear. Further enzyme engineering will effectively solve this problem and provide novel biocatalysts with higher chain-length specificity.

  - Pathway engineering

Different from the previous two engineering strategies, pathway engineering centers on optimizing and reconstructing complex biosynthetic pathways, often by introducing heterologous genes and synthetic elements. Generally, there are two primary strategies for increasing DCAs production. The first involves suppressing pathways related to FA metabolism, and the second relies on enhancing ω-oxidation through the increased expression of the pertinent enzymes (Figure 3C).

In living organisms, FAs are typically metabolized by β-oxidation for energy production rather than ω-oxidation. Since DCAs are often utilized as a carbon source for cell growth, it is crucial to tailor β-oxidation to exclude the chain-length of the substrate, thus preventing the degradation of DCAs. In Y. lipolytica, β-oxidation involves three enzymes: Pox, Pot, and Mfe. Pox is the rate-limiting enzyme catalyzing the initial step of the pathway, and has garnered significant attention. It is important to note that Y. lipolytica harbors six POX genes (POX1-6), and extensive studies have been used to test the chain-length specificity of the encoded enzymes. Wang et al. found that the POX2 gene and POX3 gene encode long-chain and short-chain oxidases, respectively (Wang et al., 1999a). To prevent adipic acid degradation, Lee et al. constructed six mutant strains through deleting each POX gene, and derived three POX double-knockout strains from the POX3-disrupted strain BSHF003. The BSHF024 strain (pox1pox3) produced the highest adipic acid titer up to 214.50 mg/L after 96 h of fermentation (Lee et al., 2024). Rational β-oxidation design based on the chain length selectivity of POX is therefore an effective strategy for the synthesis of DCAs with specific chain lengths.

Additionally, pathway engineering can target other crucial enzymes in the β-oxidation pathway downstream from Pox, such as Mfe, which, unlike Pox, does not have multiple isozymes in yeast. In Y. lipolytica, deleting the MFE2 gene boosted the formation of short-chain DCAs (C9), although the longer-chain DCAs continued to be consumed with no accumulation observed (Werner and Zibek, 2017). Continued in-depth research may lead to new metabolic engineering strategies that further enhance DCAs production by optimizing β-oxidation processes.

Enhanced DCAs production can be achieved by specifically inhibiting β-oxidation while boosting ω-oxidation. Within the latter pathway, the ω-hydroxylase complex, comprising a CYP from the CYP52 family and NADPH-dependent Cpr, catalyzes the first and most significant rate-limiting step (Figure 2). Y. lipolytica exhibits functional diversity among its CYP enzymes, containing 12 isozymes with varying substrate specificity and physiological roles (Alk1-Alk12) (Takai et al., 2012). Iwama et al. found that out of all Alk isozymes, only Alk3, Alk5, and Alk7 exhibit activity on C12 FAs, as demonstrated by individually expressing each ALK gene in a Y. lipolytica strain where all ALK genes had been deleted (Iwama et al., 2016). In addition, Cpr is responsible for electron transfer in this reaction and generally interacts with Alk. A recent study found that ALK5 overexpression resulted in the most significant increase of adipic acid production of Y. lipolytica, and proved that the introduction of CPR would further increase the product titer (Lee et al., 2024).

The second step of ω-oxidation is catalyzed by Fao or Adh, representing another rate-limiting step in the formation of DCAs. In Y. lipolytica, Fao is generally considered to play a major role in this reaction (Gatter et al., 2014). Lee et al. introduced one copy of the FAO gene into the POX1 locus of the strain (pox1pox3) through chromosomal integration to generate the new strain. In YPDP50 medium supplemented with methyl laurate, the new one produced 224.70 mg/L adipic acid, which was 46% more than the original (Lee et al., 2024). Another point of concern is that when FAs are converted into DCAs, the -oxidation leads to the oxidation of one NADPH molecule and the reduction of two NAD(P)+ molecules, which disrupts the redox balance. Sathesh-Prabu et al. (2018) found that the expression of a xylose reductase (XR) gene helped the engineered E. coli strain balance the cofactor supply, resulting in a 180% increase of the DCAs yield. Hence, cofactor regeneration is also very important for DCAs production.

The combination of - and ω-oxidation is currently the main method for the biosynthesis of DCAs from HSs. In the future, the rapid development of metabolic engineering will greatly promote the further optimization of this method.

  - Cell engineering

The engineering strategies discussed here can significantly increase DCAs productivity. Additionally, cell engineering plays a crucial role in enhancing strain stability, as it helps mitigate the stress associated with DCAs accumulation and increases the strain's resilience under strenuous fermentation conditions. This discussion will now concentrate on adaptive laboratory evolution (ALE) as a key approach for whole-cell engineering (Figure 3D).

ALE is an innovative engineering strategy that leverages the principles of natural selection, as proposed by Darwin, by applying artificial selection pressure to identify mutant strains with improved phenotypes (Chen et al., 2024). These strains often exhibit superior growth and enhanced tolerance to the target products, positioning ALE as a valuable adjunct to rational engineering strategies aimed at optimizing the balance between cellular growth and the production of high-value compounds. Furthermore, ALE influences various levels of gene and protein regulation, making it feasible to utilize omics techniques to analyze genotypic differences between evolved and parental strains. This helps elucidate genotype-phenotype relationships, providing deeper insights into gene functions and evolutionary mechanisms (Hirasawa and Maeda, 2023).

Pereira et al. used ALE to significantly improve the tolerance of S. cerevisiae to glutaric, adipic and pimelic acid, after which whole-genome sequencing revealed that the most common mechanism for enhancing DCAs tolerance was the upregulation of multidrug transporters (PDR12 and QDR3) (Pereira et al., 2019). Cui et al. performed ALE on Y. lipolytica to restore the growth of an engineered strain and achieve high production of succinic acid (Cui et al., 2023). In general, cell engineering strategies can improve the resistance of strains to DCAs, resulting in higher yields without affecting their growth.

  - Fermentation engineering

Scale-up from shake-flask to fermenter culture is a critical step in the industrialization of DCAs production. Fermentation engineering realizes high product titers by optimizing fermentation conditions and process parameters (Figure 3E).

In the bioconversion process of HSs into DCAs, a two-phase bioprocess is widely considered to be optimal (Lee et al., 2017; Lee et al., 2024), which takes advantage of a separation between growth and product synthesis. The growth phase is mainly optimized for the fast proliferation of strains. During this time, the pH is generally maintained at 5.0-5.5 (Jeon et al., 2019; Lee et al., 2019), so that the cells can make full use of glucose and other organic matter, and produce enzymes related to the generation of DCAs under the induction of n-alkanes. In the conversion phase, the pH is gradually increased to 7.5–8.0 (Lee et al., 2019), strain growth comes to halt, and HSs enter the cells for α-oxidation and -oxidation.

The expression of yeast CYP/Alk as well as Cpr can be induced by hydrocarbons and FAs, but this induction is also depend on the growth phase, the intracellular pH and oxygen availability (Liu et al., 2003; Rehm et al., 1983). Therefore, some important parameters in the fermentation process need to be optimized, such as the composition of the fermentation medium, pH, dissolved oxygen (DO) and temperature. Since HSs are not readily bioavailable, the addition of moderate emulsifiers can be used to disperse the substrate into fine particles which are more easily assimilated by microorganisms (Lee et al., 2018). Research has shown that adding H2O2 to the fermentation process can not only enhance the oxygen supply, but also induce CYP, the key enzyme of α- and -oxidation, to improve the yield of DCAs (Jiao et al., 2001). With the continuous production of DCAs, the pH gradually decreases, affecting product synthesis. Therefore, the pH of the fermentation broth should be strictly controlled to improve the yield of DCAs. Liu et al. (2004) proposed and optimized a pH control strategy for DCAs production, and found that CYP activity was the highest under the optimal pH conditions, while the DCAs concentrations obtained from n-dodecane, n-tridecane and n-tetradecane reached 166.00, 153.00 and 138.00 g/L, respectively. Since HSs need to be converted into DCAs by multi-step oxidation, it is necessary to provide a large amount of oxygen in the fermentation process to ensure the normal progress of fermentation. At the same time, the substrate feed rate is a crucial parameter for process optimization since high concentrations of the substrate can have toxic effects on the cells as well as the potential to cause foaming and fermenter overflow (Sugiharto et al., 2018). Jeon et al. (2019) maintained the pH at 5.5 in the growth stage, gradually increased it to 7.8 during the conversion stage, and successfully achieved a high yield of Sebacic acid in 5- and 50-L fermenters by controlling the substrate feed rate and DO level (Table 2).

Thus, fermentation optimization is crucial for DCAs industrialization, and along with the aforementioned fermentation engineering strategies, it is essential to systematically optimize the specific composition of the medium and fermentation conditions to achieve the maximum production of DCAs.

  - Downstream engineering

The separation and purification of DCAs from the fermentation broth is the last step of the production process. Efficient downstream processes can provide high-quality precursors for biobased materials such as nylon (Figure 3F).

The fermentation broth of DCAs is a typical four-phase system consisting of “gas-liquid (water phase)-oil (hydrophobic substrate)-solid (yeast)”, and its composition is complex, so the separation and purification of DCAs is a complex and challenging step. In this process, the reaction is usually terminated by heating the fermentation broth to induce cell lysis, after which the DCAs are converted into the corresponding sodium salts y adding concentrated sodium hydroxide. The solid and liquid phases are further separated by membrane filtration or centrifugation, the obtained liquid is decolorized with activated carbon, and the fermentation broth after decolorization is acidified and crystallized. Finally, DCAs are obtained through refining steps such as filtration, washing and drying (Gu et al., 2024).

In situ product removal (ISPR) is an extraction technique that swiftly removes target compounds without harming cells, thereby preventing the final products from inhibiting cell growth (Yu et al., 2023). In the biological fermentation process of DCAs, with the continuous increase of DCAs concentration, the toxicity of DCAs to cells is also increasing, so the emergence of ISPR can solve this problem well. A growing number of studies have demonstrated the feasibility of ISPR in producing DCAs (Cheng et al., 2012; Panduric et al., 2017; Ruebenach et al., 2024). Li et al. developed an integrated fermentation system that combines DCAs fermentation with the expanded-bed adsorption (EBA) for in-situ separation. EBA is a coupled process that combines solid-liquid separation and product recovery into a single unit operation. They employed ISPR technology to enhance cell growth during the product inhibitory period of microbial fermentation, significantly increasing the yield of DCAs, with a final titer of 145.20 g/L, an average yield of 0.52 g/g, and a productivity of 1.30 g/L/h. In contrast to the fed-batch fermentation, which yielded 73.80 g/L of DCAs over 48 hours with a total yield of 0.57 g/g glucose and productivity of 1.54 g/L/h. The ISPR approach sustains similar production efficiency and yield while significantly enhancing the ultimate DCAs concentration (Li et al., 2011). On the other hand, the rapid development of biotechnology makes the separation of DCAs easier. Wang et al. changed the morphological characteristics of the production strain by regulating the expression of related genes to facilitate cell separation, which simplified the downstream process of microbial production and ultimately achieved cost reduction (Wang et al., 2019). In general, downstream processing is still a costly item in microbial production of DCAs, and additional studies are needed to develop more cost-effective DCAs separation strategies.

- Challenges and future perspectives

Although useful engineering strategies and their corresponding challenges have been described previously, there are other obstacles in the efficient production of DCAs, which are briefly described below and provide prospects for future development.

At present, the production of DCAs is increasingly emphasizing green and efficient bioconversion. However, the bioconversion of HSs into DCAs still relies on petroleum-based feedstocks, such as alkanes, which are increasingly being depleted. Advances in synthetic biology have greatly broadened the substrate spectrum of chassis microorganisms. By reprogramming the metabolic pathways of E. coli, Li et al. finally realized the production of 15.26 g/L of medium-chain DCAs from waste cooking oil (Li et al., 2021). At the same time, FAMEs derived from vegetable oil are considered to be a suitable sustainable carbon source for the production of DCAs (Lee et al., 2019; Lee et al., 2024). Other green carbon sources, such as lignocellulose and sustainable C1 resources, require further studies before they can be considered for DCAs production.

As the DCAs “factory”, the chassis cell directly determines the efficiency of production. While C. tropicalis has been the primary strain used for bio-based DCAs production, it is a pathogenic yeast responsible for several human diseases and gene editing is often inefficient because of its diploid nature (Gu et al., 2024; O'Brien et al., 2021). By contrast, Y. lipolytica is an oleaginous yeast with generally recognized as safe (GRAS) status and has also been used in the production of DCAs due to its strong lipid accumulation capacity, unique oxidation pathway and excellent pH tolerance (Wang et al., 2023). Moreover, there is a rich molecular manipulation toolbox for Y. lipolytica, including Golden Gate, EasyClone YALI, CRISPR/Cas9, and many other methods (Egermeier et al., 2019; Holkenbrink et al., 2018). Additionally, microbial co-culture systems are another development direction for the efficient production of DCAs. They offer advantages such as a reduced metabolic burden, better balance of cofactor and energy supply, as well as greater stability and adaptability to environmental fluctuations (Bader et al., 2010; Peng et al., 2023).

The entry of HSs into the cells, which is a prerequisite for DCAs synthesis, is generally thought to occur by passive transport (free diffusion and facilitated diffusion), active transport and endocytosis (Figure 2). The utilization of FAMEs and triacylglycerols relies on the release of extracellular lipases, which hydrolyze these esters into free FAs. Y. lipolytica possesses a variety of genes encoding lipases, including members of both the LIP and TGL families (Fickers et al., 2003; Pereira-Meirelles et al., 2000). Alkane uptake may require a direct interaction between cells and alkanes droplets, which is linked to protrusion formation (Fickers et al., 2005), and cell contact can be facilitated by the secretion of biosurfactants (emulsification) (Fukuda, 2023). Emulsification is facilitated by a small extracellular glycoprotein called liposan, which is produced in response to growth in the presence of HSs (F. Thevenieau, 2010). Although the uptake of hydrophobic molecules in yeasts was initially believed to be a passive process occurring through diffusion across the cell membrane, it is now understood to be an active process mediated by proteins (Claus et al., 2019). Extracellular FAs can also be transported into Y. lipolytica through a yet unidentified transporter, referred to as eFbp1-4p (Onesime et al., 2022; Soong et al., 2023). In Y. lipolytica, the ABC transporter YlABC1-4p is thought to be involved in alkane uptake and exhibits different substrate specificities (Thevenieau et al., 2007). Additionally, Y. lipolytica as a dimorphic yeast, the change of cell morphology may affect the absorption of alkanes (Zinjarde et al., 1998). However, the precise mechanism by which hydrophobic compounds traverse the membrane is still debated, but it can be used as a starting point to increase the yield of DCAs in future studies.

In the future, more environmentally friendly carbon sources as well as more efficient and safer chassis cells should be selected. In addition, it will be necessary to optimize every step of DCAs production under the guidance of metabolic and process engineering, so that the product yield can reach an economically viable level. In short, practical DCAs bioprocess development, including scale-up from the laboratory to the factory, must ensure high DCAs titers as well as stable, controlled biological processes. The rational engineering strategies reviewed in this paper can provide a reference for these endeavors. With the development of synthetic biotechnology, industrial-scale fermentation of engineered microorganisms will become the main source of DCAs in the future.

- Conclusions

The production of medium-long chain dicarboxylic acids from renewable hydrophobic substrates using oleaginous yeasts as platform strains is in line with the concept of green and sustainable development. However, the current medium-long chain dicarboxylic acids production still cannot meet the requirements of industrialization. This paper reviewed the strategies of metabolic engineering and process optimization for improving the production of medium-long chain dicarboxylic acids, hoping to promote the development of medium-long chain dicarboxylic acids biosynthesis with the help of synthetic biology.

Declaration of Competing Interest

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

# Reference

- Ali, M., Ishqi, H.M., Husain, Q., 2020. Enzyme engineering: Reshaping the biocatalytic functions. Biotechnol. Bioeng. 117 (6), 1877-1894.

- Aziz, M., St-Louis, R., Husson, F., Kermasha, S., 2016. Selected dehydrogenases in Yarrowia lipolytica JMY 861: their role in the synthesis of flavor compounds. Biosci., Biotechnol., Biochem. 80 (11), 2184-2191.

- Bader, J., Mast-Gerlach, E., Popovic, M.K., Bajpai, R., Stahl, U., 2010. Relevance of microbial coculture fermentations in biotechnology. J. Appl. Microbiol. 109 (2), 371-387.

- Bart, J.C.J., Cavallaro, S., 2015. Transiting from Adipic Acid to Bioadipic Acid. Part II. Biosynthetic Pathways. Ind. Eng. Chem. Res. 54 (2), 567-576.

- Bell, E.L., Hutton, A.E., Burke, A.J., O'Connell, A., Barry, A., O'Reilly, E., Green, A.P., 2024. Strategies for designing biocatalysts with new functions. Chem. Soc. Rev. 53 (6), 2851-2862.

- Berchmans, H.J., Hirata, S., 2008. Biodiesel production from crude Jatropha curcas L. seed oil with a high content of free fatty acids. Bioresour. Technol. 99 (6), 1716-1721.

- Berninger, G., Schmidtchen, R., Casel, G., Knorr, A., Rautenstrauss, K., Kunau, W.H., Schweizer, E., 1993. Structure and metabolic control of the yarrowia-lipolytica peroxisomal 3-oxoacyl-Coa-thiolase gene. Eur. J. Biochem. 216 (2), 607-613.

- Bertram, J.H., Mulliner, K.M., Shi, K., Plunkett, M.H., Nixon, P., Serratore, N.A., Douglas, C.J., Aihara, H., Barney, B.M., 2017. Five Fatty Aldehyde Dehydrogenase Enzymes from Marinobacter and Acinetobacter spp. and Structural Insights into the Aldehyde Binding Pocket. Appl. Environ. Microbiol. 83 (12), e00018-17.

- Bigey, F., Pasteur, E., Polomska, X., Thomas, S., Crutz-Le Coq, A.M., Devillers, H., Neuveglise, C., 2023. Insights into the Genomic and Phenotypic Landscape of the Oleaginous Yeast Yarrowia lipolytica. J. Fungi 9 (1), 76.

- Cao, W., Li, H., Luo, J., Yin, J., Wan, Y., 2017a. High-level productivity of α,ω-dodecanedioic acid with a newly isolated Candida viswanathii strain. J. Ind. Microbiol. Biotechnol. 44 (8), 1191-1202.

- Cao, W., Liu, B., Luo, J., Yin, J., Wan, Y., 2017b. α, ω-Dodecanedioic acid production by Candida viswanathii ipe-1 with co-utilization of wheat straw hydrolysates and n-dodecane. Bioresour. Technol. 243, 179-187.

- Cazier, A.P., Blazeck, J., 2021. Advances in promoter engineering: Novel applications and predefined transcriptional control. Biotechnol. J. 16 (10), e2100239.

- Chae, T.U., Ahn, J.H., Ko, Y.S., Kim, J.W., Lee, J.A., Lee, E.H., Lee, S.Y., 2020. Metabolic engineering for the production of dicarboxylic acids and diamines. Metab. Eng. 58, 2-16.

- Chen, C., Li, Y.W., Chen, X.Y., Wang, Y.T., Ye, C., Shi, T.Q., 2024. Application of adaptive laboratory evolution for Yarrowia lipolytica: A comprehensive review. Bioresour. Technol. 391, 129893.

- Chen, X., Denning, K.L., Mazur, A., Lawrence, L.M., Wang, X., Lu, Y., 2023. Under peroxisome proliferation acyl-CoA oxidase coordinates with catalase to enhance ethanol metabolism. Free Radical Biol. Med. 208, 221-228.

- Cheng, K.K., Zhao, X.B., Zeng, J., Wu, R.C., Xu, Y.Z., Liu, D.H., Zhang, J.A., 2012. Downstream processing of biotechnological produced succinic acid. Appl. Microbiol. Biotechnol. 95 (4), 841-850.

- Claus, S., Jezierska, S., Van Bogaert, I.N.A., 2019. Protein-facilitated transport of hydrophobic molecules across the yeast plasma membrane. FEBS Lett. 593 (13), 1508-1527.

- Craft, D.L., Madduri, K.M., Eshoo, M., Wilson, C.R., 2003. Identification and characterization of the CYP52 family of Candida tropicalis ATCC 20336, important for the conversion of fatty acids and Alkanes to α,ω-dicarboxylic acids. Appl. Environ. Microbiol. 69 (10), 5983-5991.

- Cui, Z., Zhong, Y., Sun, Z., Jiang, Z., Deng, J., Wang, Q., Nielsen, J., Hou, J., Qi, Q., 2023. Reconfiguration of the reductive TCA cycle enables high-level succinic acid production by Yarrowia lipolytica. Nat. Commun. 14 (1), 8480.

- Egermeier, M., Sauer, M., Marx, H., 2019. Golden Gate-based metabolic engineering strategy for wild-type strains of Yarrowia lipolytica. FEMS Microbiol. Lett. 366 (4), fnz022.

- Elmore, C.L., Porter, T.D., 2002. Modification of the nucleotide cofactor-binding site of cytochrome P-450 reductase to enhance turnover with NADH in vivo. J. Biol. Chem. 277 (50), 48960-48964.

- Endoh-Yamagami, S., Hirakawa, K., Morioka, D., Fukuda, R., Ohta, A., 2007. Basic helix-loop-helix transcription factor heterocomplex of Yas1p and Yas2p regulates cytochrome P450 expression in response to alkanes in the yeast Yarrowia lipolytica. Eukaryotic Cell 6 (4), 734-743.

- F. Thevenieau, A.B., T. Desfougeres, J. Sabirova, K. Albertin, S. Zinjarde & J.M. Nicaud, 2010. Uptake and Assimilation of Hydrophobic Substrates by the Oleaginous Yeast Yarrowia lipolytica, in: Timmis, K.N. (Eds.), Handbook of Hydrocarbon and Lipid Microbiology. Springer, Berlin, Heidelberg, pp. 1513-1527.

- Fickers, P., Benetti, P.H., Waché, Y., Marty, A., Mauersberger, S., Smit, M.S., Nicaud, J.M., 2005. Hydrophobic substrate utilisation by the yeast Yarrowia lipolytica, and its potential applications. FEMS Yeast Res. 5 (6-7), 527-543.

- Fickers, P., Nicaud, J.M., Destain, J., Thonart, P., 2003. Overproduction of lipase by Yarrowia lipolytica mutants. Appl. Microbiol. Biotechnol. 63 (2), 136-142.

- Folz, J., Culver, R.N., Morales, J.M., Grembi, J., Triadafilopoulos, G., Relman, D.A., Huang, K.C., Shalon, D., Fiehn, O., 2023. Human metabolome variation along the upper intestinal tract. Nat. Metab. 5 (5), 777-+.

- Fukuda, H., Kondo, A., Noda, H., 2001. Biodiesel fuel production by transesterification of oils. J. Biosci. Bioeng. 92 (5), 405-416.

- Fukuda, R., 2013. Metabolism of Hydrophobic Carbon Sources and Regulation of It in n-Alkane-Assimilating Yeast Yarrowia lipolytica. Biosci., Biotechnol., Biochem. 77 (6), 1149-1154.

- Fukuda, R., 2023. Utilization of n-alkane and roles of lipid transfer proteins in Yarrowia lipolytica. World J. Microbiol. Biotechnol. 39 (4), 97.

- Funk, I., Rimmel, N., Schorsch, C., Sieber, V., Schmid, J., 2017. Production of dodecanedioic acid via biotransformation of low cost plant-oil derivatives using Candida tropicalis. J. Ind. Microbiol. Biotechnol. 44 (10), 1491-1502.

- Gallegos, J.E., Rose, A.B., 2015. The enduring mystery of intron-mediated enhancement. Plant Sci. 237, 8-15.

- Gargiulo, S., Soumillion, P., 2021. Directed evolution for enzyme development in biocatalysis. Curr. Opin. Chem. Biol. 61, 107-113.

- Gatter, M., Foerster, A., Baer, K., Winter, M., Otto, C., Petzsch, P., Jezkova, M., Bahr, K., Pfeiffer, M., Matthaeus, F., Barth, G., 2014. A newly identified fatty alcohol oxidase gene is mainly responsible for the oxidation of long-chain ω-hydroxy fatty acids in Yarrowia lipolytica. FEMS Yeast Res. 14 (6), 858-872.

- Gu, S., Zhu, F., Zhang, L., Wen, J., 2024. Mid-Long Chain Dicarboxylic Acid Production via Systems Metabolic Engineering: Progress and Prospects. J. Agric. Food Chem. 72 (11), 5555-5573.

- Guo, Y., Song, H., Wang, Z., Ding, Y., 2012. Expression of POX2 gene and disruption of POX3 genes in the industrial Yarrowia lipolytica on the γ-decalactone production. Microbiol. Res. 167 (4), 246-252.

- Haddouche, R., Delessert, S., Sabirova, J., Neuveglise, C., Poirier, Y., Nicaud, J.M., 2010. Roles of multiple acyl-CoA oxidases in the routing of carbon flow towards β-oxidation and polyhydroxyalkanoate biosynthesis in Yarrowia lipolytica. FEMS Yeast Res. 10 (7), 917-927.

- Hirakawa, K., Kobayashi, S., Inoue, T., Endoh-Yamagami, S., Fukuda, R., Ohta, A., 2009. Yas3p, an Opi1 Family Transcription Factor, Regulates Cytochrome P450 Expression in Response to n-Alkanes in Yarrowia lipolytica. J. Biol. Chem. 284 (11), 7126-7137.

- Hirasawa, T., Maeda, T., 2023. Adaptive Laboratory Evolution of Microorganisms: Methodology and Application for Bioproduction. Microorganisms 11 (1), 92.

- Holkenbrink, C., Dam, M.I., Kildegaard, K.R., Beder, J., Dahlin, J., Belda, D.D., Borodina, I., 2018. EasyCloneYALI: CRISPR/Cas9-Based Synthetic Toolbox for Engineering of the Yeast Yarrowia lipolytica. Biotechnol. J. 13 (9), e1700543.

- Hong, S.P., Seip, J., Walters-Pollak, D., Rupert, R., Jackson, R., Xue, Z., Zhu, Q., 2012. Engineering Yarrowia lipolytica to express secretory invertase with strong FBA1IN promoter. Yeast 29 (2), 59-72.

- Hung, W.H., Chen, P.K., Fang, C.W., Lin, Y.C., Wu, P.C., 2021. Preparation and Evaluation of Azelaic Acid Topical Microemulsion Formulation: In Vitro and In Vivo Study. Pharmaceutics 13 (3), 410.

- Iida, T., Sumita, T., Takagi, M., 2000. The cytochrome P450ALK multigene family of an n-alkane-assimilating yeast, Yarrowia lipolytica: cloning and characterization of genes coding for new CYP52 family members. Yeast 16 (12), 1077-1087.

- Ilchenko, A.P., Morgunov, I.G., Honeck, H., Mauersberger, S., Vasilkova, N.N., Muller, H.G., 1994. Purification and properties of alcohol oxidase from the yeast yarrowia-lipolytica h-222. BIOCHEMISTRY-MOSCOW+ 59 (9), 969-974.

- Iwama, R., Kobayashi, S., Ishimaru, C., Ohta, A., Horiuchi, H., Fukuda, R., 2016. Functional roles and substrate specificities of twelve cytochromes P450 belonging to CYP52 family in n-alkane assimilating yeast Yarrowia lipolytica. Fungal Genet. Biol. 91, 43-54.

- Iwama, R., Kobayashi, S., Ohta, A., Horiuchi, H., Fukuda, R., 2015. Alcohol dehydrogenases and an alcohol oxidase involved in the assimilation of exogenous fatty alcohols in Yarrowia lipolytica. FEMS Yeast Res. 15 (3), fov014.

- Iwama, R., Kobayashi, S., Ohta, A., Horiuchi, H., Fukuda, R., 2014. Fatty Aldehyde Dehydrogenase Multigene Family Involved in the Assimilation of n-Alkanes in Yarrowia lipolytica. J. Biol. Chem. 289 (48), 33275-33286.

- Jeon, W.Y., Jang, M.J., Park, G.Y., Lee, H.J., Seo, S.H., Lee, H.S., Han, C., Kwon, H., Lee, H.C., Lee, J.H., Hwang, Y.T., Lee, M.O., Lee, J.G., Lee, H.W., Ahn, J.O., 2019. Microbial production of sebacic acid from a renewable source: production, purification, and polymerization. Green Chem. 21 (23), 6491-6501.

- Jiao, P., Huang, Y.M., Li, S.L., Hua, Y.T., Cao, Z. 2001. Effects and mechanisms of H2O2 on production of dicarboxylic acid. Biotechnol. Bioeng. 75 (4), 456-462.

- Ju, J.H., Oh, B.R., Heo, S.Y., Lee, Y.U., Shon, J.H., Kim, C.H., Kim, Y.M., Seo, J.W., Hong, W.K., 2020. Production of adipic acid by short- and long-chain fatty acid acyl-CoA oxidase engineered in yeast Candida tropicalis. Bioprocess Biosyst. Eng. 43 (1), 33-43.

- Kim, J.Y., Jun, M.W., Seong, Y.J., Park, H., Ahn, J., Park, Y.C., 2019. Direct Biotransformation of Nonanoic Acid and Its Esters to Azelaic Acid by Whole Cell Biocatalyst of Candida tropicalis. ACS Sustainable Chem. Eng. 7 (21), 17958-17966.

- Kim, J.Y., Ahn, Y.J., Lee, J.A., Lee, S.Y., 2023. Recent advances in the production of platform chemicals using metabolically engineered microorganisms. Curr. Opin. Green Sustainable Chem. 40, 100777.

- Kim, T.H., Kang, S.H., Han, J.E., Seo, E.J., Jeon, E.Y., Choi, G.E., Park, J.B., Oh, D.K., 2020a. Multilayer Engineering of Enzyme Cascade Catalysis for One-Pot Preparation of Nylon Monomers from Renewable Fatty Acids. ACS Catal. 10 (9), 4871-4878.

- Kim, T.H., Kang, S.H., Park, J.B., Oh, D.K., 2020b. Construction of an engineered biocatalyst system for the production of medium-chain α,ω-dicarboxylic acids from medium-chain ω-hydroxycarboxylic acids. Biotechnol. Bioeng. 117 (9), 2648-2657.

- Ko, Y.S., Kim, J.W., Lee, J.A., Han, T., Kim, G.B., Park, J.E., Lee, S.Y., 2020. Tools and strategies of systems metabolic engineering for the development of microbial cell factories for chemical production. Chem. Soc. Rev. 49 (14), 4615-4636.

- Kobayashi, S., Hirakawa, K., Horiuchi, H., Fukuda, R., Ohta, A., 2013. Phosphatidic acid and phosphoinositides facilitate liposome association of Yas3p and potentiate derepression of ARE1 (alkane-responsive element one)-mediated transcription control. Fungal Genet. Biol. 61, 100-110.

- Kurihara, T., Ueda, M., Okada, H., Kamasawa, N., Naito, N., Osumi, M., Tanaka, A., 1992. Beta-oxidation of butyrate, the short-chain-length fatty-acid, occurs in peroxisomes in the yeast candida-tropicalis. J. Biochem. 111 (6), 783-787.

- Lai, J., Song, L., Zhou, Y., Zong, H., Zhuge, B., Lu, X., 2023. Fine-Tuned Gene Expression Elements from Hybrid Promoter Libraries in Pichia pastoris. ACS Synth. Biol. 13 (1), 310-318.

- Lawson, C.E., Marti, J.M., Radivojevic, T., Jonnalagadda, S.V.R., Gentz, R., Hillson, N.J., Peisert, S., Kim, J., Simmons, B.A., Petzold, C.J., Singer, S.W., Mukhopadhyay, A., Tanjore, D., Dunn, J.G., Martin, H.G., 2021. Machine learning for metabolic engineering: A review. Metab. Eng. 63, 34-60.

- Lee, H., Han, C., Lee, H.W., Park, G., Jeon, W., Ahn, J., Lee, H., 2018. Development of a promising microbial platform for the production of dicarboxylic acids from biorenewable resources. Biotechnol. Biofuels 11, 310.

- Lee, H., Sugiharto, Y.E.C., Lee, H., Jeon, W., Ahn, J., Lee, H., 2019. Biotransformation of dicarboxylic acids from vegetable oil-derived sources: current methods and suggestions for improvement. Appl. Microbiol. Biotechnol. 103 (4), 1545-1555.

- Lee, H., Sugiharto, Y.E.C., Lee, S., Park, G., Han, C., Jang, H., Jeon, W., Park, H., Ahn, J., Kang, K., Lee, H., 2017. Characterization of the newly isolated ω-oxidizing yeast Candida sorbophila DS02 and its potential applications in long-chain dicarboxylic acid production. Appl. Microbiol. Biotechnol. 101 (16), 6333-6342.

- Lee, S.M., Lee, J.Y., Hahn, J.S., Baek, S.H., 2024. Engineering of Yarrowia lipolytica as a platform strain for producing adipic acid from renewable resource. Bioresour. Technol. 391, 129920.

- Li, G., Huang, D., Sui, X., Li, S., Huang, B., Zhang, X., Wu, H., Deng, Y., 2020. Advances in microbial production of medium-chain dicarboxylic acids for nylon materials. React. Chem. Eng. 5 (2), 221-238.

- Li, Q., Wang D., Hu G., Xing J., Su Z., 2011. Integrated bioprocess for high-efficiency production of succinic acid in an expanded-bed adsorption system. Biochem. Eng. J. 56 (3), 150-157.

- Li, Y., Cheng, Z., Zhao, C., Gao, C., Song, W., Liu, L., Chen, X., 2021. Reprogramming Escherichia coli Metabolism for Bioplastics Synthesis from Waste Cooking Oil. ACS Synth. Biol. 10 (8), 1966-1979.

- Lim, S., Yoo, H.w., Sarak, S., Kim, B.g., Yun, H., 2021. A multi-enzyme cascade reaction for the production of α,ω-dicarboxylic acids from free fatty acids. J. Ind. Eng. Chem. 98, 358-365.

- Liu, S.C., Li, C., Fang, X.C., Cao, Z.A., 2004. Optimal pH control strategy for high-level production of long-chain α-,ω-dicarboxylic acid by Candida tropicalis. Enzyme Microb. Technol. 34 (1), 73-77.

- Liu, S.C., Li, C., Xie, L.Y., Cao, Z., 2003. Intracellular pH and metabolic activity of long-chain dicarboxylic acid-producing yeast Candida tropicalis. J. Biosci. Bioeng. 96 (4), 349-353.

- Liu, Y., Zhang, J., Li, Q., Wang, Z., Cui, Z., Su, T., Lu, X., Qi, Q., Hou, J., 2022. Engineering Yarrowia lipolytica for the sustainable production of β-farnesene from waste oil feedstock. Biotechnol. Biofuels Bioprod. 15 (1), 101.

- Lu, W., Ness, J.E., Xie, W., Zhang, X., Minshull, J., Gross, R.A., 2010. Biosynthesis of Monomers for Plastics from Renewable Oils. J. Am. Chem. Soc. 132 (43), 15451-15455.

- Markel, U., Essani, K.D., Besirlioglu, V., Schiffels, J., Streit, W.R., Schwaneberg, U., 2020. Advances in ultrahigh-throughput screening for directed enzyme evolution. Chem. Soc. Rev. 49 (1), 233-262.

- Mauersberger, S., Drechsler, H., Oehme, G., Muller, H.G., 1992. Substrate-specificity and stereoselectivity of fatty alcohol oxidase from the yeast candida-maltosa. Appl. Microbiol. Biotechnol. 37 (1), 66-73.

- Meinhold, P., Peters, M.W., Hartwick, A., Hernandez, A.R., Arnold, F.H., 2006. Engineering Cytochrome P450 BM3 for Terminal Alkane Hydroxylation. Adv. Synth. Catal. 348, 763–772.

- Messina, E., de Souza, C.P., Cappella, C., Barile, S.N., Scarcia, P., Pisano, I., Palmieri, L., Nicaud, J.M., Agrimi, G., 2023. Genetic inactivation of the Carnitine/Acetyl-Carnitine mitochondrial carrier of Yarrowia lipolytica leads to enhanced odd-chain fatty acid production. Microb. Cell Fact. 22 (1), 128.

- Morais, S., Knoll-Gellida, A., Andre, M., Barthe, C., Babin, P.J., 2007. Conserved expression of alternative splicing variants of peroxisomal acyl- CoA oxidase 1 in vertebrates and developmental and nutritional regulation in fish. Physiol. Genomics 28 (3), 239-252.

- Nebert, D.W., Nelson, D.R., Coon, M.J., Estabrook, R.W., Feyereisen, R., Fujiikuriyama, Y., Gonzalez, F.J., Guengerich, F.P., Gunsalus, I.C., Johnson, E.F., Loper, J.C., Sato, R., Waterman, M.R., Waxman, D.J., 1991. The P450 superfamily-update on new sequences, gene-mapping, and recommended nomenclature. DNA Cell Biol. 10 (1), 1-14.

- O'Brien, C.E., Oliveira-Pacheco, J., O Cinneide, E., Haase, M.A.B., Hittinger, C.T., Rogers, T.R., Zaragoza, O., Bond, U., Butler, G., 2021. Population genomics of the pathogenic yeast Candida tropicalis identifies hybrid isolates in environmental samples. Plos PLoS Pathog. 17 (3), e1009138.

- Onesime, D., Vidal, L., Thomas, S., Henry, C., Martin, V., Andre, G., Kubiak, P., Minard, P., Celinska, E., Nicaud, J.M., 2022. A unique, newly discovered four-member protein family involved in extracellular fatty acid binding in Yarrowia lipolytica. Microb. Cell Fact. 21 (1), 200.

- Packer, M.S., Liu, D.R., 2015. Methods for the directed evolution of proteins. Nat. Rev. Genet. 16 (7), 379-394.

- Panduric, N., Salic, A., Zelic, B., 2017. Fully integrated biotransformation of fumaric acid by permeabilized baker's yeast cells with in situ separation of L-malic acid using ultrafiltration, acidification and electrodialysis. Biochem. Eng. J. 125, 221-229.

- Park, B.G., Kim, J., Kim, E.J., Kim, Y., Kim, J., Kim, J.Y., Kim, B.G., 2021. Application of Random Mutagenesis and Synthetic FadR Promoter for de novo Production of ω-Hydroxy Fatty Acid in Yarrowia lipolytica. Front. Bioeng. Biotechnol. 9, 624838.

- Peng, H., Chen, R., Shaw, W.M., Hapeta, P., Jiang, W., Bell, D.J., Ellis, T., Ledesma-Amaro, R., 2023. Modular Metabolic Engineering and Synthetic Coculture Strategies for the Production of Aromatic Compounds in Yeast. ACS Synth. Biol. 12 (6), 1739-1749.

- Pereira-Meirelles, F.V., Rocha-Leao, M.H.M., Sant' Anna, G.L., 2000. Lipase location in Yarrowia lipolytica cells. Biotechnol. Lett. 22 (1), 71-75.

- Pereira, R., Wei, Y., Mohamed, E., Radi, M., Malina, C., Herrgard, M.J., Feist, A.M., Nielsen, J., Chen, Y., 2019. Adaptive laboratory evolution of tolerance to dicarboxylic acids in Saccharomyces cerevisiae. Metab. Eng. 56, 130-141.

- Petkevicius, K., Wenning, L., Kildegaard, K.R., Sinkwitz, C., Smedegaard, R., Holkenbrink, C., Borodina, I., 2022. Biosynthesis of insect sex pheromone precursors via engineered β-oxidation in yeast. FEMS Yeast Res. 22 (1), foac041.

- Picataggio, S., Rohrer, T., Deanda, K., Lanning, D., Reynolds, R., Mielenz, J., Eirich, L.D., 1992. Metabolic engineering of candida-tropicalis for the production of long-chain dicarboxylic-acids. Bio-Technol. 10 (8), 894-898.

- Ranea-Robles, P., Houten, S.M., 2023. The biochemistry and physiology of long-chain dicarboxylic acid metabolism. Biochem. J. 480 (9), 607-627.

- Rehm, H.J., Hortmann, L., Reiff, I., 1983. Regulation of the formation of fatty-acids at the microbial oxidation of alkanes. Acta Biotechnol. 3 (3), 279-288.

- Rios, J., Lebeau, J., Yang, T., Li, S., Lynch, M.D., 2021. A critical review on the progress and challenges to a more sustainable, cost competitive synthesis of adipic acid. Green Chem. 23 (9), 3172-3190.

- Ruebenach, L., Kruse, N., Rose, M., 2024. Separation of Biobased Carboxylic Acids by Aqueous Phase Nanofiltration. Chem. Ing. Tech, 96(4), 432-439.

- Sathesh-Prabu, C., Lee, S.K., 2018. Enhancement of α,ω-Dicarboxylic Acid Production by the Expression of Xylose Reductase for Refactoring Redox Cofactor Regeneration. J. Agric. Food Chem. 66 (13), 3489-3497.

- Sathesh-Prabu, C., Lee, S.K., 2015. Production of Long-Chain α,ω-Dicarboxylic Acids by Engineered Escherichia coli from Renewable Fatty Acids and Plant Oils. J. Agric. Food Chem. 63 (37), 8199-8208.

- Seo, E.J., Yeon, Y.J., Seo, J.H., Lee, J.H., Bongol, J.P., Oh, Y., Park, J.M., Lim, S.M., Lee, C.G., Park, J.B., 2018. Enzyme/whole-cell biotransformation of plant oils, yeast derived oils, and microalgae fatty acid methyl esters into n-nonanoic acid, 9-hydroxynonanoic acid, and 1,9-nonanedioic acid. Bioresour. Technol. 251, 288-294.

- Sequeiros-Borja, C.E., Surpeta, B., Brezovsky, J., 2021. Recent advances in user-friendly computational tools to engineer protein function. Briefings Bioinf. 22 (3), bbaa150.

- Soong, Y.H.V., Coleman, S.M., Liu, N., Qin, J., Lawton, C., Alper, H.S., Xie, D., 2023. Using oils and fats to replace sugars as feedstocks for biomanufacturing: Challenges and opportunities for the yeast Yarrowia lipolytica. Biotechnol. Adv. 65, 108128.

- Strijbis, K., Distel, B., 2010. Intracellular Acetyl Unit Transport in Fungal Carbon Metabolism. Eukaryotic Cell 9 (12), 1809-1815.

- Sugiharto, Y.E.C., Lee, H., Fitriana, A.D., Lee, H., Jeon, W., Park, K., Ahn, J., Lee, H., 2018. Effect of decanoic acid and 10-hydroxydecanoic acid on the biotransformation of methyl decanoate to sebacic acid. Amb Express 8, 75.

- Sun, M.L., Shi, T.Q., Lin, L., Ledesma-Amaro, R., Ji, X.J., 2022. Advancing Yarrowia lipolytica as a superior biomanufacturing platform by tuning gene expression using promoter engineering. Bioresour. Technol. 347, 126717.

- Takai, H., Iwama, R., Kobayashi, S., Horiuchi, H., Fukuda, R., Ohta, A., 2012. Construction and characterization of a Yarrowia lipolytica mutant lacking genes encoding cytochromes P450 subfamily 52. Fungal Genet. Biol. 49 (1), 58-64.

- Thevenieau, F., Le Dall, M.T., Nthangeni, B., Mauersberger, S., Marchal, R., Nicaud, J.M., 2007. Characterization of Yarrowia lipolytica mutants affected in hydrophobic substrate utilization. Fungal Genet. Biol. 44 (6), 531-542.

- Thomas Beardslee, S.P., 2012. Bio-based adipic acid from renewable oils. Lipid Technol. 24 (10), 223-225.

- Valm, A.M., Cohen, S., Legant, W.R., Melunis, J., Hershberg, U., Wait, E., Cohen, A.R., Davidson, M.W., Betzig, E., Lippincott-Schwartz, J., 2017. Applying systems-level spectral imaging and analysis to reveal the organelle interactome. Nature 546 (7656), 162-+.

- Victorino da Silva Amatto, I., Gonsales da Rosa-Garzon, N., Antonio de Oliveira Simoes, F., Santiago, F., Pereira da Silva Leite, N., Raspante Martins, J., Cabral, H., 2022. Enzyme engineering and its industrial applications. Biotechnol. Appl. Biochem. 69 (2), 389-409.

- Waché, Y., Aguedo, M., LeDall, M.T., Nicaud, J.M., Belin, J.M., 2002. Optimization of Yarrowia lipolytica's β-oxidation pathway for γ-decalactone production. J. Mol. Catal. B Enzym. 19, 347-351.

- Wan, Y., Lee, J.M., 2021. Toward Value-Added Dicarboxylic Acids from Biomass Derivatives via Thermocatalytic Conversion. ACS Catal. 11 (5), 2524-2560.

- Wang, H.J., Le Dall, M.T., Waché, Y., Laroche, C., Belin, J.M., Nicaud, J.M., 1999a. Cloning, sequencing, and characterization of five genes coding for Acyl-CoA oxidase isozymes in the yeast Yarrowia lipolytica. Cell Biochem. Biophys. 31 (2), 165-174.

- Wang, H.J.J., Le Dall, M.T., Waché, Y., Laroche, C., Belin, J.M., Gaillardin, C., Nicaud, J.M., 1999b. Evaluation of acyl coenzyme A oxidase (Aox) isozyme function in the n-alkane-assimilating yeast Yarrowia lipolytica. J. Bacteriol. 181 (17), 5140-5148.

- Wang, J., Peng, J., Fan, H., Xiu, X., Xue, L., Wang, L., Su, J., Yang, X., Wang, R., 2018. Development of mazF-based markerless genome editing system and metabolic pathway engineering in Candida tropicalis for producing long-chain dicarboxylic acids. J. Ind. Microbiol. Biotechnol. 45 (11), 971-981.

- Wang, K., Lin, L., Wei, P., Ledesma-Amaro, R., Ji, X.J., 2023. Combining orthogonal plant and non-plant fatty acid biosynthesis pathways for efficient production of microbial oil enriched in nervonic acid in Yarrowia lipolytica. Bioresour. Technol. 378, 129012.

- Wang, Y., Ling, C., Chen, Y., Jiang, X., Chen, G.Q., 2019. Microbial engineering for easy downstream processing. Biotechnol. Adv. 37 (6), 107365.

- Werner, N., Zibek, S., 2017. Biotechnological production of bio-based long-chain dicarboxylic acids with oleogenious yeasts. World J. Microbiol. Biotechnol. 33 (11), 194.

- Xie, W.J., Warshel, A., 2023. Harnessing generative AI to decode enzyme catalysis and evolution for enhanced engineering. Natl. Sci. Rev. 10 (12), nwad331.

- Xiong, X., Chen, S., 2020. Expanding Toolbox for Genes Expression of Yarrowia lipolyticato Include Novel Inducible, Repressible, and Hybrid Promoters. ACS Synth. Biol. 9 (8), 2208-2213.

- Xu, D., Ma, C., Wu, M., Deng, Y., He, Y.C., 2023. Improved production of adipic acid from a high loading of corn stover via an efficient and mild combination pretreatment. Bioresour. Technol. 382, 129196.

- Yamagami, S., Morioka, D., Fukuda, R., Ohta, A., 2004. A basic helix-loop-helix transcription factor essential for cytochrome P450 induction in response to alkanes in yeast Yarrowia lipolytica. J. Biol. Chem. 279 (21), 22183-22189.

- Ye, L., Yang, C., Yu, H., 2018. From molecular engineering to process engineering: development of high-throughput screening methods in enzyme directed evolution. Appl. Microbiol. Biotechnol. 102 (2), 559-567.

- Ye, M., Gao, J., Li, J., Yu, W., Bai, F., Zhou, Y.J., 2024. Promoter engineering enables precise metabolic regulation towards efficient β-elemene production in Ogataea polymorpha. Synth. Syst. Biotechnol. 9 (2), 234-241.

- Yu, J.L., Xia, X.X., Zhong, J.J., Qian, Z.G., 2017. Enhanced production of C5 dicarboxylic acids by aerobic-anaerobic shift in fermentation of engineered Escherichia coli. Process Biochem. 62, 53-58.

- Yu, S., Cui, J., Wang, X., Zhong, C., Li, Y., Yao, J., 2020. Preparation of Sebacic Acid via Alkali Fusion of Castor Oil and its Several Derivatives. J. Am. Oil Chem. Soc. 97 (6), 663-670.

- Yu, Y., Zhou, Y., Wang, K., Sun, T., Lin, L., Ledesma-Amaro, R., Ji, X.J., 2023. Metabolic and Process Engineering for Producing the Peach-Like Aroma Compound γ-Decalactone in Yarrowia lipolytica. J. Agric. Food Chem. 71 (1), 110-120.

- Zhang, M., Li, Q., Fan, F., Lu, F., Zhang, X., 2023. Combinatorial metabolic engineering and tolerance evolution of Yarrowia lipolytica for improved γ-decalactone synthesis. Process Biochem. 132, 248-254.

- Zhao, B., Zhang, Y., Wang, Y., Lu, Z., Miao, L., Wang, S., Li, Z., Sun, X., Han, Y., He, S., Zhang, Z., Xiao, D., Zhang, C., Foo, J.L., Wong, A., Yu, A., 2023. Biosynthesis of a-bisabolene from low-cost renewable feedstocks by peroxisome engineering and systems metabolic engineering of the yeast Yarrowia lipolytica. Green Chem. 25 (20), 8145-8159.

- Zhou, P., Gao, C., Song, W., Wei, W., Wu, J., Liu, L., Chen, X., 2024. Engineering status of protein for improving microbial cell factories. Biotechnol. Adv. 70, 108282.

- Zinjarde, S.S., Pant, A., Deshpande, M.V., 1998. Dimorphic transition in Yarrowia lipolytica isolated from oil-polluted sea water. Mycol. Res. 102, 553-558.

Table 1. Production of medium-long chain dicarboxylic acids from different carbon sources

| Features | Hydrophobic substrates | Hydrophilic substrates |
| --- | --- | --- |
| Chassis cell | Oleaginous yeasts (Yarrowia lipolytica, Candida tropicalis) | Model microorganisms (Saccharomyces cerevisiae, Escherichia coli) |
| Clear genetic background |  |  |
| Abundance of genetic elements |  |  |
| Ease of gene editing |  |  |
| Simple metabolic pathways |  |  |
| Degree of structural similarity to dicarboxylic acids |  |  |
| Substrate toxicity to cell |  |  |
| Oxygen requirement |  |  |
| Fermentation yield |  |  |
| Degree of commercialization |  |  |
| Composition of fermentation system | Four phases: gas-liquid (water phase)-oil (hydrophobic substrate)-solid (yeast) | Three phases: gas-liquid (water phase)-solid (cells) |
| The pH shift during fermentation | Two-stage fermentation requires to increase pH | It is generally not necessary to adjust the pH |

The number of asterisks represents the relative level of development for different carbon sources; maximum: , intermediate: , minimum: .

# Table 2. Summary of studies on dicarboxylic acid production by microorganisms using hydrophobic substances.

| Dicarboxylic acid | Host | Substrate | Cultivation | Titer (g/L) | Productivity (g/L/h) | Yield (g/g) | Metabolic engineering strategies | Ref. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Adipic acid | Candida tropicalis | Methyl laurate | 5 L, fed-batch fermentation | 12.10 | 0.10 | NAa | Deletion of AOX4; overexpression of AOX5 | (Ju et al., 2020) |
|  | C. tropicalis | Fatty acids mixture (coconut oil) | Fed-batch fermentation | 50.00 | 0.38 | NA | Deletion of POX4; overexpression of key enzymes of ω-oxidation | (Thomas Beardslee, 2012) |
|  | Yarrowia lipolytica | Methyl laurate | Shake flask, batch | 1.18 | NA | NA | Deletion of POX1 and POX3; introduction of an additional copy of ALK5-CPR1 with FAO1 | (Lee et al., 2024) |
| Sebacic acid | C. tropicalis | Methyl decanoate | 5 L, continuous fermentation | 34.50 | 0.64 | NA | NA | (Sugiharto et al., 2018) |
|  | C. tropicalis | Decanoic acid methyl ester | 5 L, fed-batch fermentation | 98.30 | 0.57 | 0.98b | Overexpression of CYP52A19, CPR, and FAO1 | (Jeon et al., 2019) |
|  | C. tropicalis | Decanoic acid methyl ester | 50 L, fed-batch fermentation | 90.90 | 0.68 | 0.98b | Overexpression of CYP52A19, CPR, and FAO1 | (Jeon et al., 2019) |
| Undecanedioic acid | Escherichia coli | Ricinoleic acid | Shake flask, batch | 53.60 | 1.49 | 0.83b | Introduction of ADH, BVMO, MBP, ALDH | (Kim et al., 2020a) |
| Dodecanedioic acid | Y. lipolytica | Dodecane | 600 mL, fed-batch fermentation | 11.00 | NA | NA | Deletion of POX1-6 and overexpression of FAO1 | (Gatter et al., 2014) |
|  | Candida viswanathii | Dodecane | 7.5 L, fed-batch fermentation | 181.60 | 1.59 | NA | NA | (Cao et al., 2017a) |
|  | C. viswanathii | Dodecane | 7.5 L, fed-batch fermentation | 129.70 | 1.13 | NA | NA | (Cao et al., 2017b) |
|  | C. tropicalis | Dodecanoic acid methyl ester | 1 L, fed-batch fermentation | 66.00 | 0.35 | 0.80 | NA | (Funk et al., 2017) |
|  | Wickerhamiella sorbophila | Methyl laurate | 5 L, fed-batch fermentation | 92.50 | 0.83 | NA | Deletion of POX1 | (Lee et al., 2018) |
|  | C. viswanathii | n-Dodecane | 3 L, fed-batch fermentation | 224.00 | 1.87 | 0.83b | Overexpression of CYP52A19, CPRb, FAO, and POS5 | (Pham et al., 2023) |
|  | C. tropicalis | n-Dodecane | 5 L, batch fermentation | 166.00 | NA | NA | NA | (Liu et al., 2004) |
|  | C. tropicalis | n-Dodecane | 15 L, fed-batch fermentation | 140.00 | 0.90 | 0.80b | Deletion of POX4 and POX5 | (Picataggio et al., 1992) |
| Tridecanedioic acid | C. tropicalis | n-Tridecane | 5 L, batch fermentation | 153.00 | NA | NA | NA | (Liu et al., 2004) |
| Tetradecanedioic acid | C. tropicalis | n-Tetradecane | 5 L, batch fermentation | 138.00 | NA | NA | NA | (Liu et al., 2004) |
|  | C. tropicalis | Methyl tetradecanoate | 15 L, fed-batch fermentation | 210.00 | 1.30 | 1.00b | Deletion of POX4 and POX5 | (Picataggio et al., 1992) |

aThese values were not mentioned in the main text of the original reference. bMolar conversion.

# Figures:

Figure 1. Overview of the production of dicarboxylic acids (DCAs) from different carbon sources and their applications. The abbreviations are as follows: FAMEs, fatty acid methyl esters; FAs, fatty acids.

Figure 2. The metabolic process of producing dicarboxylic acids (DCAs) from hydrophobic substances in oleaginous yeasts. Firstly, hydrophobic substrates cross the cell membrane through several mechanisms. Then, the hydrophobic substrates undergo α- and -oxidation to form DCAs inside the cell, followed by the degradation of DCAs in peroxisomes via -oxidation. Finally, the acetyl-CoA produced via -oxidation directly enters the mitochondria as acetyl-carnitine through the carnitine acyltransferase (CAT) shuttle system. The abbreviations are as follows: ABC1-4, n-alkane transporters; ACS, acyl-CoA synthetase; ADH, fatty alcohol dehydrogenase; CPR, NADPH-dependent cytochrome P450 reductase; CYP/ALK, cytochrome P450 monooxygenase; eFBP1-4, FA transporter; ER, endoplasmic reticulum; FALDH, fatty aldehyde dehydrogenase; FAMEs, fatty acid methyl esters; FAO, fatty alcohol oxidase; FAs, fatty acids; MFE, multifunctional enzyme; MIT, mitochondria; POT, 3-ketoacyl-CoA thiolase; POX, acyl-CoA oxidase. The green box illustrates the enzymes that participate in -oxidation; the purple box illustrates the enzymes that participate in α-oxidation; the blue box illustrates the enzymes that participate in -oxidation; the gray dashed line indicates the presumed DCAs production process.

Figure 3. Metabolic and process engineering strategies for producing dicarboxylic acids (DCAs) in oleaginous yeasts. (A): promoter engineering strategy, achieving precise gene regulation by adjusting promoter strength; (B): enzyme engineering strategy, mining new enzymes by taking advantage of bioinformatics approaches, enhancing enzyme activity or catalytic efficiency via random mutagenesis or rational design; (C): pathway engineering strategy, achieving high production of target DCAs by enhancing -oxidation and specifically regulating -oxidation; (D): cell engineering strategy, improving the stability and productivity of strains via adaptive laboratory evolution; (E): fermentation engineering strategy, improving the yield of DCAs by two-phase bioprocesses and optimization of fermentation conditions; (F): downstream engineering strategy, increasing product purity using effective separation systems.

Figure 1

Figure 2

Figure 3

Graphical abstract