Estimating particulate exposure from modern Municipal Waste Incinerators (MWIs) in Great Britain

Philippa Douglasa, Anna Freni Sterrantinoa, Maria Leal Sancheza, Danielle C Ashwortha, Rebecca E Ghosha, Daniela Fechta, Anna Fontb, Marta Blangiardoa, John Gullivera, Mireille B Toledanoa, Paul Elliotta, Kees de Hoogha,c,d, Gary W Fullerb, and Anna L Hansella,e*

- UK Small Area Health Statistics Unit, MRC-PHE Centre for Environment and Health, Imperial College London, London, United Kingdom

- Environment Research Group, MRC-PHE Centre for Environment and Health, Imperial College London, London, United Kingdom

- Swiss Tropical and Public Health Institute, Basel, Switzerland

- University of Basel, Basel, Switzerland

- Directorate of Public Health and Primary Care, Imperial College Healthcare NHS Trust, London, United Kingdom

*Corresponding author

UK Small Area Health Statistics Unit (SAHSU), Department of Epidemiology and Biostatistics

School of Public Health, Imperial College London

St Mary’s Campus, Norfolk Place, W2 1PG

a.hansell@imperial.ac.uk

+44 (0)20 7594 3344

Abstract

Municipal Waste Incineration (MWI) is regulated through the European Union Directive on Industrial Emissions (IED), but there is ongoing public concern regarding potential hazards to health. Using dispersion modelling, we estimated spatial variability in PM10 concentrations arising from MWIs at postcodes (average 12 households) within 10km of MWIs in Great Britain (GB) in 2003-10. We also investigated change points in PM10 emissions in relation to introduction of EU Waste Incinerator Directive (EU-WID) (subsequently transposed into IED) and correlations of PM10 with SO2, NOx, heavy metals and polychlorinated dibenzo-p-dioxins/furans (PCDD/Fs) emissions.

Yearly average modelled PM10 concentrations were 1.00 10-5 to 5.53 10-2 µg m-3; a small contribution to ambient background levels which were typically 6.59 – 2.68 101 µg m-3, 3-5 orders of magnitude higher. While low, concentration surfaces are likely to represent a spatial proxy of other relevant pollutants. There were statistically significant correlations between PM10 and heavy metal compounds (other heavy metals (r=0.43, p=<0.001)), polycyclic aromatic hydrocarbons (PAHs) (r=0.20, p=0.050) and polychlorinated biphenyls (PCBs) (r=0.19, p-0.022). No clear change points were detected following EU-WID implementation, possibly as incinerators were operating to EU-WID standards before the implementation date. Results will be used in an epidemiological analysis examining potential associations between MWIs and health outcomes.

Keywords

Municipal Waste Incineration, exposure assessment, particulate matter, air quality, dispersion modelling

Introduction

Incineration of domestic and commercial waste is increasing in Europe in response to European Union (EU) legislation to divert waste from landfill. Waste incinerator feedstock includes paper, food, plastics, glass, electrical appliances and other non-hazardous materials and may vary day to day and from incinerator to incinerator1. Composition of combustion emissions depends on feedstock mix but potentially comprises particulate matter, sulphur dioxide (SO2), nitrogen oxides (NOx), hydrogen chloride (HCl), carbon monoxide (CO), Volatile Organic Compounds (VOCS), Persistent Organic Pollutants (POPs) such as polychlorinated dibenzo-p-dioxins/furans (PCDD/Fs), polychlorinated biphenyls (PCBs), polycyclic aromatic hydrocarbons (PAHs); and heavy metals.1,2 Limits for incinerator emissions are set by the EU. The Waste Incineration Directive (EU-WID) (2000/76/EC) was implemented in Great Britain (GB) on 28 December 2002 for new municipal waste incinerators (MWIs) and on 28 December 2005 for existing facilities. In 2010 the EU-WID was transposed into the Industrial Emissions Directive (IED) (2010/75/EU) which combined several directives; this was implemented in GB in 2013 for new MWIs and 2014 for existing facilities.3,4

While there is public concern regarding potential adverse health effects from MWI emissions, findings from epidemiological studies are inconsistent and inconclusive.5 Most studies have focused on adult cancers, 6–9 and to a lesser extent reproductive and child health outcomes.10–15 Exposure assessment has often used simple proxies, adopting proximity to incinerator as the exposure measure.16–19 There have been three recent studies of incinerators conducted in Italy. One used a modified risk-assessment model to estimate lung cancer risk .35 The remaining two used a dispersion model to assess exposure to particulate matter with a diameter <10 µm (PM10) and found a higher risk of miscarriage14 and pre-term delivery15 with increasing PM10 exposure (but no associations with sex ratio, multiple births, or frequency of small for gestational age15), where estimated PM10 levels from incinerators were consistent with those estimated near two British incinerators.20

Particulate matter/total dust emissions are monitored continuously and reported as daily means as part of the EU-WID regulations, so that dispersion of these emissions can be modelled in areas near incinerators on a daily basis (whereas heavy metals, PCDD/Fs, PAHs, PCBs are measured periodically3 to check compliance). We previously reported20 methods for dispersion modelling around two MWIs. In the present study, our main aim was to model the spatial distribution of PM10 concentrations within 10 km of GB MWIs in operation 2003-10 for the resident population. We were also able to look at whether there were:

- Emissions above the EU-WID daily average particulate (total dust) limit value of 10 mg m-3 per flue.4

- Correlations and associations between PM10 emissions and within-flue emission measurements of heavy metals, PCDD/Fs, PAHs and PCBs (to provide information about the chemical composition of PM10 being emitted from flues).

- Changes in levels of PM10 emissions after the implementation of the EU-WID, which reduced daily average emission limit values from 30 mg m-3 to 10 mg m-3 for particulate matter/total dust.21

We refer to PM10 rather than total suspended particulates (total dust) throughout as size fraction studies have found all particulate incinerator emissions are <10 µm diameter.22

Materials and Methods

Study area

We included all 22 MWIs in Great Britain (Figure 1) in operation between 2003-10, comprising 14 existing MWIs and 8 new MWIs opening after 2003. We excluded one MWI in the Isle of Man (Richmond Hill) for which we did not have health or emissions data and three other incinerators open during this time as they were not solely MWIs (Fawley, Hampshire; Ellesmere Port, Cheshire ; Peak Load Boiler, Shetland). The location of MWIs varied as to the geographical characteristics of their location and topography. A 10 km radius around each MWI was chosen as the study area as per Ashworth et al.20

Incinerator Data

Information on the emissions, total annual licensed throughput, the number of flues, and whether an MWI opened to or adopted EU-WID specifications were provided by the Environment Agency (EA), Natural Resources Wales and the Scottish Environment Protection Agency (SEPA) (Table 1). Information on characteristics used in dispersion modelling including; height and diameter of the MWI stack (m), exit temperature (°C), flow rate (m s-1), and velocity (m s-1) per MWI per flue are reported in Supporting Information (SI) A, Table S1. Daily measured PM10 emissions per MWI, per flue and per year (some originally in paper format, which were digitised and quality checked by a third party) were provided by the EA and SEPA. Non-numeric and negative values were recoded according to an algorithm agreed with the EA and SEPA (SI B, Table S2).

MWIs varied in size and location (Figure 1). The licensed throughput varied from 3,500 (Porthmellon) to 750,000 (Edmonton) tonnes per annum (Table 1). Populations living within 10 km of each MWI varied from 2,203 (Porthmellon) to 2,726,145 (SELCHP) (information from census data from 2011). The majority of MWIs had multiple flues (15 MWIs; 12 with two flues, three with three flues). The number of non-operational and missing days varied from MWI to MWI and from year to year, and occurred sporadically for a few days, or for longer periods (e.g. several months; Table 1 and SI E Table S4)).

The availability of heavy metals, PCDD/Fs, PAHs and PCB measurements varied. MWIs are required to complete up to four measurements per annum.23 Typically in-flue measurements of 6-8 heavy metals and 3-4 PCDD/F, PAH and PCBs are completed per year, per MWI and per flue, but repeated measurements are taken if higher than limit values. Heavy metals, PCDD/Fs, PAHs and PCBs were monitored in-flue, usually at the same time as each other over an eight hour period, using European committee standards (CEN).

Dispersion modelling to estimate spatial distribution of PM10 concentrations within 10 km of GB MWIs

The Atmospheric Dispersion Modelling System Urban (ADMS-Urban) (version 2.3), utilised in previous studies characterising emissions from MWIs,14,15,20,24,25 was used to model ground-level PM10 concentrations for postcode (average 12 household per postcode) area centroids within a 10 km radius of the MWIs. ADMS-Urban is a Gaussian based dispersion model that has been widely used and extensively validated. The model characterises the atmospheric boundary layer using the Monin-Obukhov length and boundary layer depth. It is capable of simulating the effects of plume rise, and the effects of buildings and complex topography on dispersion.26

The parameterisation of the dispersion model is described in Ashworth et al.20 In brief, all MWIs were modelled as point sources. Locations were verified using site addresses, grid references and aerial photography. The EU-WID requires that average emission values are reported after subtraction of a fixed amount (taken as 30%) to account for measurement instrument uncertainty.4 The EU-WID also allows up to ten measured daily average values to be discarded per year if there has been measurement instrument calibration or maintenance. For the purposes of the dispersion modelling, the emissions data were therefore increased by 30% from the reported values provided (except for Isle of Wight MWI, which did not subtract 30% to account for measurement instrument uncertainty). For the purposes of assessing emissions above the EU-WID daily average, emission values with 30% subtracted were used, as this is how compliance is assessed against the EU-WID limits. Missing emissions data were imputed using the median PM10 value of the operational days for each year and each MWI (for justification of this approach see SI D; for counts of days with missing data per flue/year/MWI see SI E). For Coventry, Dudley and Kirklees MWIs, data were missing for entire year(s), and therefore it was not possible to model PM10 dispersion for these years. Hourly meteorological data from meteorological stations within 30 km of each MWI were obtained, and selected for use based on land cover (to ensure that the land type surrounding the meteorological station was representative of the MWI) and completeness of data (for choice of meteorological station and justification see SI F). The Monin-Obukhov (MO) length (the height at which turbulence is driven by buoyancy instead of wind) and surface roughness (SR) at the dispersion site was computed by ADMS-Urban based on input meteorological variables and surface information extracted from CORINE land cover data from 2000 as per Ashworth et al.20 (SI G, Table S6 contains MO and SR values). Complex terrain was included in the dispersion modelling if more than 5% of the area surrounding a MWI (within 10 km) contained slopes of 10% or higher, which was assessed using Ordnance Survey PANORAMA data. Daily average PM10 concentrations were calculated at every postcode area centroid within 10 km of each MWI. Where the 10 km radius around MWIs overlapped (Edmonton and SELCHP in London; Tyseley, Dudley and Wolverhampton in the Midlands) the modelled output concentrations were summed for each day. Our dispersion modelling was specific for MWIs and did not account for sources of PM10 around the incinerators.

It was not practical to include the effects of buildings within the ADMS-Urban model as the study area was large (10km radius from each MWI). Moreover the MWI stack height was high (median = 72.5m, max = 100m), taller than surrounding buildings, and thus nearby buildings would not affect dispersion patterns. Instead different MO and SR lengths for each MWI were used to represent different land-uses around each MWI and effects on pollutant turbulence and dispersion (see SI G). It was not considered possible to model spatial distributions of PCDD/F, PCB, PAH or heavy metals using flue emissions data due to the sparseness and variability of the measurements.

Correlations and associations between PM10 emissions and other flue emissions

Pair-wise correlation was used to evaluate correlations of heavy metal compounds, PCDD/Fs, PAHs and PCBs and daily averages of SO2, NOx and PM10 measured during the same time period. Measurements of heavy metals for most incinerators were reported as Cd and Tl and their compounds (CdTl), Hg and its compounds (HgComp) and grouped other heavy metals (OHMs) comprising Sb, As, Cr, Pb, Co, Cu, Mn, Ni, and V (see SI C, Table S3). As the data were not normally distributed, a nonparametric Spearman’s rank correlation was used. This produces a coefficient, r, which ranges from -1 to 1. Values of -1 and 1 represent perfect negative or positive correlation respectively, whereas a value of 0 represents no correlation. A Spearman correlation p-value <0.05 was considered  statistically significant. As a Spearman’s rank correlation will not account for differences in MWI operations, flues, and years of data, a linear multiple regression model was used to adjust for these factors; we considered one pollutant at a time and used PM10, year, flues and MWI as predictors. Data were checked for normality (from Q-Q plots) and log transformed if necessary. We report the estimated coefficients with p-values, and the partial η2 -which is the variance associated with an effect divided by that variance plus the error variance, to describe the proportion of variance accounted for by the variable.

Detecting changes in levels of PM10 emissions after the implementation of the EU-WID

Data from MWI installations operating prior to the EU-WID were investigated to determine if emissions changed before or after the implementation date and when any change took place, as timings could be used to inform epidemiological analyses investigating changes in health outcomes rates before/after EU-WID implementation. Firstly, we conducted a descriptive analysis of the daily average modelled concentrations (Table 2, SI H, Figure S3) over time. Secondly, as MWIs had implemented technical changes at unspecified time points leading up to the EU-WID implementation date of 28 December 2005, we carried out a retrospective change point identification analysis using a Batch Change Detection (BCD) approach, which attempts to identify periods when there are changes in the time series distribution. We were unable to use time trend analysis due to gaps in the data from non-operational days and missing data. For the BCD analysis we used monitored in-flue 2003-10 PM10 emissions data for 11 of the 14 existing MWIs in operation when the EU-WID was adopted (Table 1). Dundee MWI was excluded as data were only available from 2005, Dudley MWI and Kirklees MWI were excluded as data were missing for 2003-05 and the third flue for Stockton-on-Tees was excluded as it was only operational in 2009 and 2010 (see SI E, Table S4). Coventry MWI could only be considered from 2004 onwards as data for 2003 were missing (see SI E, Table S4). Daily in-flue PM10 emissions data were treated as independent observations. Only days when the incinerator was classed as operational and reported daily PM10 in-flue concentrations were included. The date of the change point τ was estimated by a non-parametric Cramer-von-Mises test (details in SI I, Equation S1). This test was chosen as it suits non-Gaussian distributed data, characterised by zero inflation and by extreme values. It tests for shifts in scale (variability measures, e.g. variance) or location (e.g. mean, median). We computed a test value for each observation and defined the change point as the maximum test value. The statistical analyses were conducted in R using the cpm package.27

Results

Dispersion modelling to predict spatial distribution of PM10 concentrations within 10 km of GB MWIs

Annual mean average modelled PM10 concentrations (based on daily average modelled PM10 concentrations) per MWI and per year ranged from 1.00·10-5 to 5.53·10-2 µg m-3 (Table 2). Complex overlapping dispersion patterns were shown for those areas with overlapping fields from multiple MWIs (Figure 2): surfaces (a) SELCHP and Edmonton and (b) Dudley, Tyseley and Wolverhampton. In some instances (429 days total, 0-83 days per incinerator), a modelled output value was not calculated (across all postcodes) by ADMS-Urban even though the MWI was classified as being operational (‘on’ or ‘missing’). This may occur for a number of reasons, but is commonly due to data processing error (e.g. when wind speed values are very low (<0.75 m s-1 when measured at 10 m above ground level), or missing meteorological data.28

Emissions above the EU-WID daily average particulate limit value

There were a small number of days with emissions above the EU-WID daily average particulate limit value in 14 of the 22 MWIs, the majority of which were <20 mg m-3 (Table 1; SI J Table S7). There was no distinct pattern that might indicate that there were fewer emissions above the EU-WID daily average particulate limit value after the implementation of the EU-WID, or more instances of emissions >10 mg m-3 before it.

Correlations and associations between PM10 emissions and other flue emissions

Statistically significant correlations were observed between PM10 and OHMs (r=0.43, p<0.001), PAHs (r=0.20, p=0.050), and PCBs (r=0.19, p=0.022), with borderline statistically significant correlations between PM10 and CdTl (r=0.14, p=0.065) and PCDD/F (r=0.15, p=0.052) (Table 3).  Statistically significant estimated coefficients from the multiple linear regression were observed between PM10 and CdTl (β = 0.264, p =< 0.001), OHMs (β = 0.305, p =< 0.001), PCDD/Fs (β = 0.120, p = 0.008) and PCBs (β = 0.084, p = 0.045) (Table 4). The partial η2 are reported as percentages and these showed modest overall explanatory power of the model variance after adjusting for MWI, flue and year for CdTl (η2=4.99), NOx (η2=5.46) and OHMs (η2=14.25).

Detecting changes in levels of PM10 emissions following the EU-WID

The descriptive analysis of the average modelled concentrations showed no clear pattern of a reduction in PM10 concentrations after the implementation of the EU-WID in MWIs adopting EU-WID specifications (Table 1, SI H, Figure S3), possibly as incinerators were already complying with EU-WID standards by the implementation date which required them to fit bag filters. These descriptive findings were supported by findings in the Batch Change Detection statistical analyses (Table 5). In six of the 11 MWIs investigated, the change point occurred within a year prior or posterior to the EU-WID implementation in at least one flue (Coventry, Edmonton, Sheffield, Stockton-on-tees, Stoke-on-Trent, and Tyseley), and lower median PM10 emissions were detected after the change point in five of these (in seven out of eleven flues). However, in the remaining five MWIs the change point date was detected more than one year prior or posterior to the EU-WID implementation date (Bolton, Eastcroft, Porthmellon, SELCHP and Wolverhampton) and a higher median level of PM10 was detected after the change point date in three of these MWIs (Eastcroft, Porthmellon, SELCHP), although still within EU-WID limits (Table 5). Figures showing monitored PM10 data, with the change points for each MWI included in the analysis are presented in the supplementary information SI K (Figure S4).

Discussion

Our study of all 22 British MWIs in operation 2003-10 indicates very low concentrations of incinerator-related PM10 within 10 km of the MWIs at postcode level (annual mean concentrations ranging from 1.00 10-5 to 5.53 10-2 µg m-3). There were statistically significant correlations of PM10 emissions of heavy metals, PAHs and PCBs (r = 0.19-0.43). A change point was detected in 6 of 11 MWIs adopting EU-WID specifications within a year prior or posterior to EU-WID implementation, but statistical analyses did not provide clear evidence of major changes in (the low levels of) incinerator-related PM10 concentrations after implementation.

Dispersion modelling to estimate concentrations of PM10 within 10 km of MWIs

The annual mean modelled PM10 concentrations from GB MWIs ranged from 1.00 10-5 to 5.53 10-2 µg m-3 within a 10 km radius of the MWI. These contribute a small proportion of  UK PM10 background levels, which range between 6.59 – 2.68 101 µg m-3 (annual UK means per postcode in 2010, based on modelled data)29. As all European incinerators operate to the EU WID, this suggests that MWIs also make a small contribution to European background concentrations within 10 km of incinerators across Europe (measured ambient mean concentrations, typically in the range 2.00 101 – 5.00 101 µg m-3).30 It is recognised that dispersion modelling is a simplification of reality. ADMS-Urban is a well validated, widely used dispersion model, and model errors were reduced as much as possible by using the most complete data available, and by completing a series of sensitivity analyses to ensure that model inputs best represented actual conditions (see SI D, F and G). Missing data were imputed using median values for the particular year, informed by results from a sensitivity analysis (SI D). For three incinerators in 2005, over two thirds of data were missing (Coventry, Stoke-on-Trent; Wolverhampton (SI E)), although annual mean modelled concentrations were still within similar ranges compared to other MWIs (Table 2). We do not know reasons for missing data, but this may represent maintenance periods in 2005 to ensure the MWI complied with the WID implemented at the end of that year.

All MWIs used moving grate technologies except for Allington and Newlincs (which used fluidised bed and rotary kiln technologies respectively).31 Nixon et al.31 found that plants using fluidised bed and rotary kiln technologies had higher emissions of HCl and CO. We found no differences in PM10 emissions from Allington (mean (standard deviation (SD) 2.64 (3.60), median 1.00) and Newlincs (mean (SD) 2.44 (1.27), median 2.30) compared with the remaining 20 MWIs (mean (SD) 2.13 (2.16), median 1.60).

Results for GB incinerators are consistent with two studies conducted in Italy (also operating to the EU-WID) by Candela et al. 14 15 and the previous study conducted in GB by Ashworth et al.20 Concentration estimates were larger in Font et al. (3.00 10-2 to 1.20 10-1 µg m-3),32 however they did not measure PM10 directly but used tracers of heavy metals to estimate maximum ambient PM10 from two MWIs. The Font et al.32 study used Cd measured during plume grounding as a quantitative tracer for PM10 by multiplying measured ground-level Cd concentrations by representative in-flue PM10:Cd emission ratios. This approach set out to find a maximum value by assuming that all Cd was from the MWI.32 Our findings for PM10 are in agreement with studies on ultrafine particles involving measurements within MWI flues and ambient air, showing that incinerators do not have significant impacts on ultrafine particles in localities near MWIs.33

Additional work was undertaken to confirm the plausibility of the very low modelled PM10 concentrations. MWI emissions were fingerprinted using daily in-flue PM10 to NOX concentrations and ratios were compared to data from 15 ambient monitoring sites within 10 km of four MWIs (Edmonton, SELCHP, Tyseley and Wolverhampton) (SI L, Table S10). Results showed that whilst there was some evidence of NOx and PM10 emissions from MWIs being detected at ground-level, these were few and often could not be distinguished from other sources such as traffic (SI L, Figure S5). This supports the very low contributions of MWI PM10 to background concentrations in areas near MWIs in the present study.

Exposure surfaces for selected MWIs in Figure 2 and that have been previously presented20 show that incinerator-related PM10 concentrations were not merely a function of distance from incinerator, but showed complex spatial patterns including differences between years, largely relating to differences in emission rates (including off days) and meteorology.

Consideration of other pollutants emitted from MWIs

While ambient PM10 has been associated with adverse birth outcomes,34 levels are much higher than those arising from MWIs emissions. Despite this, some recent epidemiological studies relating to MWIs operating to the EU-WID have found associations with adverse birth outcomes.14,15 If these are causal associations, it is likely to be due to agents other than PM10 that are also emitted from incinerators such as PCDD/Fs, PAHs and heavy metals.  We were unable to model spatial distribution of these other agents directly due to sparse emissions data. Other potential incinerator emissions including polybrominated or mixed polybrominated/polychlorinated dibenzo-p-dioxins/furans (PBDD/Fs and PXDD/Fs) were not measured. However, it is a reasonable assumption that modelled spatial distribution of PM10 reflects exposure patterns of other MWI emissions. This assumption has been used in previous dispersion modelling studies, which found that heavy metals15 had a similar deposition distribution to PM10. Ranzi et al.25 measured various pollutants including sulphur oxides, nitrogen dioxide and heavy metals in Italy at maximum and minimum fallout points estimated by dispersion models and considered heavy metals as the tracer pollutant from MWIs. We found some support for this as we detected significant correlations for in-flue measurements between PM10 and heavy metals, PAHs and PCBs, which provides some support for using PM10 as a tracer.  While statistically significant, the amount of variance accounted for (partial η2) was modest, which is likely due to variability in incinerator feedstock, especially differing amounts of electrical equipment. Information on feedstock mix is not recorded by MWIs.

The level of population exposure to metals and other agents from MWIs is likely to be small.  Font et al.32 compared heavy metal emission ratios  with those measured at nearby ambient metal monitoring sites around six MWIs in England, and found limited evidence that emissions from MWIs reached ground-level.

Emissions above the EU-WID daily average particulate limit value

Although emissions greater than the EU-WID limit of 10 mg m-3 were found in 14 of the 22 MWIs, these were usually <20 mg m-3 (SI J Table S7). These may not all represent exceedances under the WID as in the event of temporary abatement failure MWIs are allowed to operate for up to 4 hours at a time (maximum 60 hours per flue per year) at an elevated half-hourly particulate limit value of 150 mg m-3 (normally 30 mg m-3). If there are less than 43 half-hourly monitoring results available in a day the daily average can be disregarded. Daily average emissions >20 mg m-3 were infrequent, and there were only rare occurrences >30 mg m-3, which may have occurred due to ‘one off’ changes in feedstock or failure of abatement systems. We were not provided with information on reasons for emissions greater than the EU-WID limit. However, given that mean PM10 concentrations estimated by the dispersion model were small (1.00 10-5 to 5.53 10-2 µg m-3; a small contribution to ambient background levels which were typically 6.59 – 2.68 101 µg m-3), these infrequent emissions above EU-WID limits would still be expected to result in very low population exposures.

Detecting changes in emissions following the EU-WID

We conducted the change point analysis for existing incinerators using the Cramer-von-Mises method to account for the ordered data structure. A simpler test (e.g. a two sample t-test to compare PM10 emissions before and after the EU-WID implementation) may have introduced bias due to the number of non-operational, missing days and non-Gaussian distributed data. We assumed that a fall in emissions would be detected in existing incinerators within one year (prior or posterior) of the EU-WID implementation date, but this was only seen for five of 11 incinerators in the change point analysis. A possible explanation is that many existing MWIs may have already met (or been modified to meet) the EU-WID requirements. However information as to whether and when each MWI adopted a new abatement system was not available. In three of the six MWIs where a change point was detected within a year prior or posterior to the EU-WID implementation date (28 December 2005), a higher mean level of PM10 was detected after the change point date in at least one flue (Edmonton, Sheffield and Tyseley), though the increases after the change point date were small, and remained below EU-WID limits. This could be related to a number of factors including differences in the feedstock or changes in the amount of waste processed over time. Since we could not identify a clear date after which emissions fell in relation to the EU-WID in pre-existing MWIs, we conclude it is not possible to conduct before/after epidemiological studies examining impact of the EU-WID on rates of adverse health outcomes  in pre-existing MWIs. However, in MWIs opening after 28 December 2002 (n=8 in 2003-10) that have always operated to the EU-WID standards, it is possible to use the opening date of the incinerator as the before/after change point date.

Overall this study suggests that PM10 exposures related to MWI emissions in Great Britain are extremely low (annual means ranging from 1.00·10-5 to 5.53·10-2 µg m-3) especially when compared to annual mean background concentrations (typically ranging between 2.00 101 – 5.00 101 µg m-3 in Europe).30 The results of the modelling will be used in an epidemiological analysis examining associations between MWIs and potential reproductive and other health effects.

Acknowledgements

The study is funded by a grant from Public Health England (PHE), by a grant from the Scottish government, funding from the MRC-PHE Centre for Environment and Health and funding from the National Institute for Health Research Health Protection Research Unit (NIHR HPRU) in Health Impact of Environmental Hazards at King’s College London and Imperial College London in partnership with Public Health England (PHE). The work of the UK Small Area Health Statistics Unit is funded by Public Health England as part of the MRC-PHE Centre for Environment and Health, funded also by the UK Medical Research Council (MR/L01341X/1).

The views expressed are those of the author(s) and not necessarily those of the NIHR, the Department of Health or Public Health England. Danielle C Ashworth was funded by a MRC PhD studentship. PE is Director of the MRC-PHE Centre for Environment and Health and a National Institute for Health Research (NIHR) senior investigator emeritus and acknowledges support from the NIHR Biomedical Research Centre at Imperial College Healthcare NHS Trust and Imperial College London. This work used the computing resources of the UK MEDical BIOinformatics partnership - aggregation, integration, visualisation and analysis of large, complex data (UK MED-BIO) which is supported by the Medical Research Council (MR/L01632X/1).

We thank the Environment Agency, Scottish Environment protection Agency (SEPA), and Natural Resources Wales for the incinerator emissions data and for their technical input. We would also like to thank the study scientific advisory committee for their valuable comments.

Supporting Information Available

(A) Municipal Waste Incinerator (MWI) characteristics, (B) Non-numeric and negative PM10 emissions value coding rules, (C) Number of times metals and other compounds were measured per pollutant per MWI, (D) Sensitivity analysis: Missing data imputation methods, (E) Number of operational, non-operational and missing days per MWI, per flue per year, (F) Meteorological data selection, (G) Monin-Obukhov and Surface Roughness length input values, (H) Mean modelled PM10 concentrations (µg m-3) per MWI that adopted EU-WID specifications, (I) Change point analysis equation, (J) Emissions above the EU-WID daily average particulate limit value, (K) Change point analysis results, (L) Fingerprinting NOx to PM10 ratios from MWI in-flue concentrations

References

(1) 	EMEP/EEA. EMEP/EEA air pollutant emission inventory guidebook 2013: Technical guidance to prepare national emission inventories - 2013. http://www.eea.europa.eu/publications/emep-eea-guidebook-2013 (accessed Sep 09, 2016)

(2) 	The Royal Commission on Environmental Pollution. Incineration of Waste. http://webarchive.nationalarchives.gov.uk/20110322143804/http:/www.rcep.org.uk/reports/index.htm (accessed Aug 8, 2015)

(3) 	Environmental permitting guidance: waste incineration - GOV.UK https://www.gov.uk/government/uploads/system/uploads/attachment_data/file/211852/pb13897-ep-core-guidance-130220.pdf (accessed Jan 29, 2016).

(4) 	The European Parliament and the Council of the European Union. 2010/75/EU http://eur-lex.europa.eu/LexUriServ/LexUriServ.do?uri=OJ:L:2010:334:0017:0119:en:PDF (accessed May 25, 2016).

(5) 	Ashworth, D. C.; Elliott, P.; Toledano, M. B. Waste incineration and adverse birth and neonatal outcomes: a systematic review. Environ. Int. 2014, 69, 120–132.

(6) 	Elliott, P.; Shaddick, G.; Kleinschmidt, I.; Jolley, D.; Walls, P.; Beresford, J.; Grundy, C. Cancer incidence near municipal solid waste incinerators in Great Britain. Br J Cancer 1996, 73 (5), 702–710.

(7) 	Elliott, P.; Eaton, N.; Shaddick, G.; Carter, R. Cancer incidence near municipal solid waste incinerators in Great Britain. Part 2: histopathological and case-note review of primary liver cancer cases. Br J Cancer 2000, 82 (5), 1103–1106.

(8) 	Porta, D. Systematic review of epidemiological studies on health effects associated with management of solid waste. Environ. Heal. 2009, 8 (1), 60.

(9) 	Reeve, N. F.; Fanshawe, T. R.; Keegan, T. J.; Stewart, A. G.; Diggle, P. J. Spatial analysis of health effects of large industrial incinerators in England, 1998–2008: a study using matched case–control areas. BMJ Open 2013, 3 (1), e001847.

(10) 	Vinceti, M.; Malagoli, C.; Teggi, S.; Fabbi, S.; Goldoni, C.; De Girolamo, G.; Ferrari, P.; Astolfi, G.; Rivieri, F.; Bergomi, M. Adverse pregnancy outcomes in a population exposed to the emissions of a municipal waste incinerator. Sci. Total Environ. 2008, 407 (1), 116–121.

(11) 	Vinceti, M.; Malagoli, C.; Fabbi, S.; Teggi, S.; Rodolfi, R.; Garavelli, L.; Astolfi, G.; Rivieri, F. Risk of congenital anomalies around a municipal solid waste incinerator: a GIS-based case-control study. Int. J. Health Geogr. 2009, 8 (1), 8.

(12) 	Ashworth, D. C.; Elliott, P.; Toledano, M. B. Waste incineration and adverse birth and neonatal outcomes: a systematic review. Environ. Int. 2014, 69, 120–132.

(13) 	Cordier, S.; Chevrier, C.; Robert-Gnansia, E.; Lorente, C.; Brula, P.; Hours, M. Risk of congenital anomalies in the vicinity of municipal solid waste incinerators. Occup. Environ. Med. 2004, 61 (1), 8–15.

(14) 	Candela, S.; Bonvicini, L.; Ranzi,  a.; Baldacchini, F.; Broccoli, S.; Cordioli, M.; Carretta, E.; Luberto, F.; Angelini, P.; Evangelista, A.; et al. Exposure to emissions from municipal solid waste incinerators and miscarriages: A multisite study of the MONITER Project. Environ. Int. 2015, 78, 51–60.

(15) 	Candela, S.; Ranzi, A.; Bonvicini, L.; Baldacchini, F.; Marzaroli, P.; Evangelista, A.; Luberto, F.; Carretta, E.; Angelini, P.; Sterrantino, A. F.; et al. Air pollution from incinerators and reproductive outcomes: a multisite study. Epidemiology 2013, 24 (6), 863–870.

(16) 	Cordioli, M.; Ranzi, A.; Leo, G. A. De; Lauriola, P. A Review of Exposure Assessment Methods in Epidemiological Studies on Incinerators. J. Environ. Public Health. 2013, Article ID 129470. DOI 10.1155/2013/129470.

(17) 	Tango, T.; Fujita, T.; Tanihata, T.; Minowa, M.; Doi, Y.; Kato, N.; Kunikane, S.; Uchiyama, I.; Tanaka, M.; Uehata, T. Risk of Adverse Reproductive Outcomes Associated with Proximity to Municipal Solid Waste Incinerators with High Dioxin Emission Levels in Japan. J. Epidemiol. 2004, 14 (3), 83–93.

(18) 	ten Tusscher, G. W.; Stam, G. A.; KOPPE, J. G. Open chemical combustions resulting in a local increased incidence of orofacial clefts. Chemosphere 2000, 40 (9-11), 1263–1270.

(19) 	Obi-Osius, N.; Misselwitz, B.; Karmaus, W.; Witten, J. Twin Frequency and Industrial Pollution in Different Regions of Hesse, Germany. Occup. Environ. Med. 2004, 61 (6), 482–487.

(20) 	Ashworth, D. C.; Fuller, G. W.; Toledano, M. B.; Font, A.; Elliott, P.; Hansell, A. L.; de Hoogh, K. Comparative assessment of particulate air pollution exposure from municipal solid waste incinerator emissions. J. Environ. Public Health. 2013. Article ID, 560342. DOI 10.1155/2013/560342.

(21) 	The European Parliament and the Council of the European Union. 2000/76/EC http://eur-lex.europa.eu/legal-content/EN/TXT/PDF/?uri=CELEX:32000L0076&from=EN (accessed May 25, 2016).

(22) 	Buonanno, G.; Ficco, G.; Stabile, L. Size distribution and number concentration of particles at the stack of a municipal waste incinerator. Waste Manag. 2009, 29 (2), 749–755.

(23) 	DEFRA. Environmental permitting guidance: waste incineration - GOV.UK https://www.gov.uk/government/publications/environmental-permitting-guidance-the-waste-incineration-directive/environmental-permitting-guidance-waste-incineration (accessed Jan 29, 2016).

(24) 	Lin, C. M.; Li, C. Y.; Mao, I. F. Birth outcomes of infants born in areas with elevated ambient exposure to incinerator generated PCDD/Fs. Environ. Int. 2006, 32 (5), 624–629.

(25) 	Ranzi, A.; Fano, V.; Erspamer, L.; Lauriola, P.; Perucci, C.; Forastiere, F. Mortality and morbidity among people living close to incinerators: a cohort study based on dispersion modeling for exposure assessment. Environ. Heal. 2011, 10 (1), 22.

(26) 	CERC. ADMS-Urban model http://www.cerc.co.uk/environmental-software/ADMS-Urban-model.html (accessed Jan 29, 2016).

(27) 	Ross, G. Parametric and nonparametric sequential change detection in R: The cpm package. J. Stat. Softw. 2015, 66 (3), 1-20.

(28) 	CERC. ADMS-Urban User guide http://www.cerc.co.uk/environmental-software/assets/data/doc_userguides/CERC_ADMS-Urban4.0_User_Guide.pdf (accessed Jun 1, 2016).

(29) 	Department for Environment, Food & Rural Affairs (Defra) Modelled background pollution data – Defra UK https://uk-air.defra.gov.uk/data/pcm-data (Accessed Aug 8, 2016)

(30) 	European Environment Agency. Particulate Matter (PM10): annual mean concentrations in Europe — European Environment Agency http://www.eea.europa.eu/themes/air/interactive/pm10#tab-based-on-data (accessed Feb 5, 2016).

(31) 	Nixon, J.D.; Wright, D.G.; Dey, P.K.; Ghosh, S.K.; Davies, P.A. A comparative assessment of waste incinerators in the UK. Waste Manage. 2013, 33, 2234-2244.

(32) 	Font, A.; de Hoogh, K.; Leal-Sanchez, M.; Ashworth, D. C.; Brown, R. J. C.; Hansell, A. L.; Fuller, G. W. Using metal ratios to detect emissions from municipal waste incinerators in ambient air pollution data. Atmos. Environ. 2015, 113, 177–186.

(33)	Jones, A.M.; Harrison, R.M. Emissions of ultrafine particles from the incineration of municpal solid waste: A review. Atmos. Environ.2016, 140, 519-528.

(34) 	Parker, J. D.; Rich, D. Q.; Glinianaia, S. V.; Leem, J. H.; Wartenberg, D.; Bell, M. L.; Bonzini, M.; Brauer, M.; Darrow, L.; Gehring, U.; et al. The international collaboration on air pollution and pregnancy outcomes: Initial results. Environ. Health Perspect. 2011, 119 (7), 1023–1028.

(35)	Scungio, M.; Buonanno, G.; Stabile, L,; Ficco, G. Lung cancer risk assessment at receptor site of a waste-to-energy plant. Waste Manage. 2016, 56, 207-215.